MCU FERTI

(V6.01)

Programming guide

This document may not be copied or distributed by any method whatsoever without written permission.

While care has been taken to ensure accuracy we cannot assume responsibility for the possible consequences of any errors that may be contained herein.

This document does not constitute any form of agreement or contract.

STATEMENT OF "CE" COMPLIANCE

The undersigned manufacturer:

ELOTEC EURL    Boulevard du Midi    47160 DAMAZAN

hereby declares that the computer product  manufactured and supplied under the registered trademark of:

MCU PLUS

complies with:

the regulations defined by European Directive 89/336/CEE

Statement made at :	DAMAZAN

On:			Monday, 31/January/2000

By: 			Mr. ERIC LONCKE

Sole proprietor and manager

TABLE OF CONTENTS

1	Control panel and main menu	1–1

1.1	POWER UP DISPAY	1–1

1.2	SOFTWARE CONFIGURATION DISPLAY	1–2

1.3	MAIN MENU DISPLAY	1–3

1.4	MENU SYSTEM	1–4

2	System parameters	2–1

2.1	GENERAL CONFIGURATION	2–1

2.1.1	Controller options	2–2

	Unit number	2–2

	MCUFerti name	2–2

	PIA Alarm	2–2

	Fixed times mem. store	2–2

	Acquisition cycle time	2–2

	Load history to PC	2–2

	Type of solarimeter	2–2

2.1.2	Copy  settings between Groups	2–3

2.1.3	Printing main Group settings	2–3

2.1.4	System date and time	2–3

	Latitude	2–4

	Longitude	2–4

	D_GMT	2–4

	Enable daylight saving	2–4

	Date summertime	2–4

	Date winter time	2–4

	Changeover time	2–4

2.1.5	System access codes	2–4

	Level "x" code	2–5

	Ref. level	2–5

	Time-out (min)	2–5

3	SETTING UP GROUPS	3–1

3.1	INTRODUCTION	3–1

3.2	EC / PH SETPOINTS	3–2

	pH setpoint	3–2

	Ec setpoint	3–2

	Inlet Ec setpoint	3–2

	Inlet Ec offset	3–2

	Inlet Ec influence	3–3

3.2.1	Hydroponic option	3–3

	Stop Ec regulation	3–3

	Stop pH regulation	3–3

3.3	NUTRIENT QUANTITIES	3–4

	Nutrient percentage	3–4

3.4	EC/PH ALARMS	3–5

	Ec tolerance	3–5

	Max Ec tolerance	3–5

	Ec monitor difference	3–5

	Ec alarm delay	3–5

	pH tolerance	3–5

	Max pH tolerance	3–5

	pH monitor difference	3–6

	pH alarm delay	3–6

3.5	INFLUENCES ON EC	3–6

	Energy influence range max.	3–6

	Energy influence range min.	3–6

	Max. Ec modification	3–6

3.6	VALVE PARAMETERS	3–7

3.6.1	Time based valve control	3–7

	Irrigation time	3–7

	Wetting time	3–8

	Max. pre-mix time	3–8

	Level reduction time	3–8

	Filling time	3–8

	Sensor rinse time	3–8

	Rinse time	3–8

	Wetting valve	3–8

	Rinsing valve	3–8

3.6.2	Volume based valve control	3–9

	Irrigation volume	3–9

	Wetting volume	3–9

	Max. pre-mix time	3–9

	Level reduction volume	3–9

	Filling time	3–9

	Sensor rinse time	3–10

	Rinsing volume	3–10

	Wetting valve	3–10

	Rinsing valve	3–10

3.6.3	Selecting valves to a group	3–10

	Val	3–10

	% column	3–10

3.6.4	Option: ‘Influence Ec per valve’	3–11

	% column	3–11

3.6.5	Valve physical parameters	3–12

	l/h/Dp	3–12

	No. dp	3–12

	m2	3–12

	St	3–12

	Drn	3–12

	Drn Dp	3–12

3.6.6	General valve setpoints	3–13

	Valve control mode	3–13

	No. simultaneous valves	3–13

	Valve interval	3–13

	Flow alarm delay	3–13

	Flow tolerance	3–13

	Flow alarm	3–14

	Minimum flow	3–14

	Tank empty aspi input	3–14

	Min. clean water time	3–14

3.6.7	Flow alarm diagrams	3–15

3.7	START MODE SETTINGS	3–17

3.7.1	Watering at fixed times of day	3–17

	Time  ( 00h00  23h59 )	3–17

3.7.2	Cyclic watering	3–19

	Cyclic start interval	3–19

	Activate start mode	3–19

	End start mode	3–19

3.7.3	Contact started watering	3–19

	Contact start interval	3–20

	Contact number	3–20

	Activation threshold	3–20

	Activate start  mode	3–20

	Deactivate start mode	3–20

3.8	OTHER IRRIGATION START MODES	3–21

3.8.1	Solar energy start mode	3–21

	Solar energy start interval	3–21

	Min. intensity level	3–21

	Energy start threshold	3–22

	Activate start mode	3–22

	End start mode	3–22

	Daily carry over	3–22

	Cumulative starts limit	3–22

	Accumulate before mode active	3–22

3.8.2	Start mode links	3–22

3.9	FROST CONTACT START MODE	3–23

	Contact start interval	3–23

	Activation threshold	3–23

	Number of contact 1	3–23

	Number of contact 2	3–23

3.9.1	Stopping a frost contact started group	3–23

3.9.2	Frost starting with 'Multifunction Contacts'	3–25

3.10	GENERAL SETTINGS	3–26

3.10.1	Periods (available as an option)	3–26

3.10.2	Programming outputs	3–28

3.10.3	Activating a group	3–29

	Weekly calendar	3–29

	Day interval	3–29

3.10.4	More general settings	3–30

	Group name	3–30

	Start interval	3–30

	Pause on Ec/pH alarm	3–30

	Irrigation priority	3–30

	Type of start contact	3–30

	Contact function	3–31

	Type of 'stop' contact	3–31

	Contact number	3–31

	Delay before contact active	3–31

	Type of 'suspend' contact	3–31

	Contact number	3–31

	Type of 'next valve' contact	3–31

	Contact number	3–31

	Resistive contact inversion	3–31

	Activation threshold	3–31

	Regulate pH for premix	3–31

	Start time	3–31

3.11	PROPORTIONAL INJECTION FUNCTION	3–32

3.11.1	Principle of operation	3–32

3.11.2	Settings menus	3–32

	Enable external injection	3–32

	Theoretical flow rate	3–32

	Irrigation volume per litre injected	3–33

	Minimum injection time	3–33

	Number of attempts before alarm	3–33

	Output number to activate	3–33

3.11.3	External injection readings	3–34

3.11.4	History	3–35

3.12	DIRECT INJECTION WITH FLOW VARIATION	3–36

3.12.1	Principle of operation	3–36

3.12.2	Calculating the flow rates for the control graph	3–37

3.12.3	Programming dilution and flow parameters	3–38

	Nutrient dilution	3–38

	Dilution Ec	3–38

	Nutrient Ai : Flow rate	3–38

3.12.4	Programming station flow variation tolerances	3–39

	Flow variation tolerance	3–39

	Flow influence delay	3–39

	Min flow for influence	3–39

	Type of flow influence	3–40

	Save initial injection time	3–40

3.13	VALVE IN CART MODE FUNCTION	3–41

3.13.1	Introduction	3–41

3.13.2	Configuring the carts	3–41

	Cart	3–41

	Contact	3–41

	Time	3–41

3.13.3	Selecting the group valves	3–43

4	MONITORING	4–1

4.1	STATUS READINGS	4–1

4.2	EXTERNAL CONTACT READINGS	4–2

4.2.1	Periodic printout	4–2

	Printout cycle	4–2

	Archive interval	4–2

4.3	STATION READINGS	4–3

4.3.1	More station readings	4–4

4.4	SOLAR ENERGY READINGS	4–4

4.5	PROGRAMMED WATERING EVENTS READOUT	4–5

4.5.1	Readout of other programmed watering modes	4–5

4.6	HISTORY	4–6

4.6.1	Group running totals and averages	4–6

4.6.2	Water starts	4–6

4.6.3	Readout of water starts by valve	4–7

5	ALARMS	5–1

5.1	MAIN DISPLAY	5–1

5.1.1	Causes of alarms	5–1

5.2	MESSAGE ALARMS	5–3

5.2.1	Causes of message alarms	5–3

6	Manual commands	6–1

6.1	OPERATION	6–1

6.2	PROGRAMMING A one-OFF WATERING START	6–2

7	Drainage	7–1

7.1	CONFIGURING THE MEASURING LINES	7–1

	Type	7–2

	K factor (i)	7–2

	K factor exponent (ii)	7–2

	Vol per pulse (i)	7–2

	Vol per pulse (ii)	7–2

	Ec - pH	7–2

7.2	SELECTING VALVES TO DRAINAGE LINES	7–2

7.3	DRAINAGE SETTINGS	7–3

	Drainage rate	7–3

	Max. solar energy influence	7–3

	Min. solar energy influence	7–3

	Max. drainage rate modification	7–3

	Min / max drainage Ec - pH	7–3

7.4	SUMMARY OF THE DRAINAGE RATE CALCULATION	7–4

7.5	DRAINAGE READINGS	7–5

7.5.1	Group readings	7–5

7.5.2	Drainage line readings	7–6

7.6	DRAINAGE INFLUENCES ON WATERING	7–7

	Rad. correction	7–7

	Contact correction.	7–7

	Cyclic correction	7–7

	Max &  min radiation	7–8

	Max & min contact	7–8

	Max & min cyclic	7–8

7.6.1	Conclusion	7–8

8	STATION WATER PRE-HEATING	8–1

8.1	HEATING SETTINGS	8–1

	Enable water heating	8–1

	Min. watering temp. for alarm	8–1

	Irrigation water temp.	8–1

	Max. watering temp. for alarm	8–2

	Min. heating water temp.	8–2

	Max. heating water temp.	8–2

	Alarm delay time	8–2

8.2	CONFIGURATION	8–2

	Heating on station number	8–2

	3WV control interval	8–2

	Stop heating if no irrigation	8–2

	Water temperature difference 3WV control	8–3

	Minimum 3WV control time per degree C	8–3

	Minimum temperature difference 3WV control	8–3

	Maximum 3WV control time per degree C	8–3

	Maximum temperature difference 3WV control	8–3

	3WV overall control time	8–4

	Maximum change in position of 3WV	8–4

	Time maintain V3V position	8–4

	Difference in position of 3WV for safety	8–4

	Pump stop delay	8–4

8.3	READINGS	8–5

9	PRINT MENUS	9–1

9.1	PROGRAMMED SETTINGS	9–1

	Print	9–1

	Group settings	9–1

	Station settings	9–1

	Valve settings	9–1

	Drainage settings	9–1

	Heating settings	9–1

9.2	CONFIGURING STATION PRINTOUTS	9–2

9.3	CONFIGURING GROUP PRINTOUTS	9–2

9.4	AUTOMATIC PRINTOUTS	9–3

	Automatic printouts	9–3

	Time of printout	9–3

	Printout of station	9–3

	Printout of groups	9–3

	Printout of valves	9–3

	Printout of drainage	9–3

	Printout of nutrients	9–3

10	INSTALLATION PARAMETERS	10-1

10.1	SYSTEM / INSTALL MENUS	10-1

10.2	INPUT / OUTPUT TEST DISPLAYS	10-2

10.2.1	Inputs display	10-2

10.2.2	Testing outputs	10-3

10.2.3	Output states	10-3

10.3	CONFIGURING GROUPS	10-4

10.4	GROUP REGULATION PARAMETERS	10-5

	Station	10-5

	Kp Ec	10-6

	KpR Ec	10-6

	Kp Ec tolerance	10-6

	Ki Ec	10-6

	Kd Ec	10-6

	Min injection time Ec	10-6

	Max injection time Ec	10-6

	Initial injection time Ec	10-6

	Injection time Ec prereg	10-7

	Kp Ecp	10-7

	Kd Ecp	10-7

	Kp pH	10-7

	KpR pH	10-7

	Kp pH tolerance	10-7

	Ki pH	10-7

	Kd pH	10-7

	Min injection time pH	10-7

	Max injection time pH	10-7

	Initial injection time pH	10-7

	Ec/pH pre-mix offset	10-7

	Ec pH maintain time	10-7

	Modify Ec pH station ON	10-7

10.5	IRRIGATION TEST MENUS	10-8

10.6	STATION CONFIGURATION	10-9

10.6.1	Flow meters	10-9

	Type	10-9

	K factor (i)	10-9

	K factor exponent (ii)	10-9

	Vol per pulse (i)	10-10

	Vol per pulse (ii)	10-10

10.6.2	Station inputs	10-10

	Input xxx	10-10

10.7	GENERAL STATION PARAMETERS	10-11

	pH regulation type	10-11

	Min. stop time	10-11

	No. valves per group	10-11

	Automatic restart	10-11

	Power supply alarm	10-11

	XXXX alarm delay	10-11

	Solarimeter gain	10-11

	3W valve inlet change over	10-11

10.8	DEFINING GROUP AND STORED CONTACTS	10-12

	St	10-12

	Cnt	10-12

	Type	10-12

	Thresh	10-12

	Inver	10-13

10.9	MULTIFUNCTION CONTACTS	10-13

# Control panel and main menu

LCD display

Function keys

(F1 to F5)

Numeric keypad

Direction arrows

Misc. keys

8

7

9

4

5

6

1

2

3

-

0

.









A















All programming and review functions of the MCU controller are accessed through the control panel keys and liquid crystal display. Function keys F1 to F5 allow you to navigate through the sequence of menus, and to confirm or abandon any modifications to the settings. The direction arrows are for moving around within the current menu, and the keypad on the left allows entry of numeric values. Three miscellaneous keys allow you to:

– cycle through characters A to Z

A

– backspace delete the last character in the current field



– copy the contents of the previous data field into the current field.



## POWER UP DISPAY

| M.C.U. Fertilization |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date : 01/07/97 Initialization : H.OK DG 0 Time last shut down : 00:00:00 Date last shut down : 00/00/00 |  |  |  |  |  |  |  |  |  |
| VER |  |  |  |  |  |  |  |  | ACCEPT |

This display includes the current date and time together with details of when the unit was last shut down (de-powered).

On the initialization line ‘H.OK’ indicates the state of the internal, back-up battery, and the letters DG refer to the software protection key (or "Dongle"), where ‘0’ after DG means the key is present. (Note that a  value of –1 after DG means the key is missing, in which case all MCUFerti control functions will be inhibited).

## SOFTWARE CONFIGURATION DISPLAY

VER

| MCUFerti version 5.04 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Prog no. : xxxx/xx Key no. : Fx.xxx Config : 8G/16V/ 0A/ 0C/ 0D/1P + PC Station : 1 2 Type : V4 A0 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | QUIT |

The first line of the display indicates the software reference number together with that of the protection key allocated to the unit.

The configuration line ("Config") indicates, respectively, the number of :

- - groups (G)

- - valves (V)

- - supplementary auxiliaries (A)

- - external contacts (C)

- - drainage lines (D)

- - periods (maximum) (P).

The letters PC signify that the system is able to communicate with a PC equipped with an RS232 port and running MCUFerti for Windows® graphical software.

Lines 3 and 4 refer to the type of fertigation stations controlled by the MCU. Each station is described by a letter :

- I	= IWAKI,

- V	= VENTURI,

- P	= PLUS/MINUS,

- A	= IRRIGATION,

followed by a figure indicating the number of nutrient injection lines fitted to the station.

## MAIN MENU DISPLAY

| Main menu |  |  |  |  |  |  |  | 00:00:00 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ALARMS |  | READOUT |  | GROUPS |  | SYSTEM |  |  |  | MANUAL |

The main menu provides the route to each of the following:

| Alarms | Display alarms and messages generated by the system An alarm can be cleared by restarting the station once any fault has been rectified. |
| --- | --- |
| Readout | Display information concerning current watering (station status), scheduled water starts, running totals and averages since midnight, and sunlight measurements (if the installation is equipped with a solarimeter). |
| Groups | Set up the parameters for the irrigation groups you need and specify the exact conditions for watering. |
| System | Configure the system (date, time, access level codes,…); use the copy and print options; set up the parameters that configure the installation (for installers). |
| Manual | Manually control watering (Start, Stop, ...), and program one or more single water starts for specific times. |

## MENU SYSTEM

ALARMS

READOUT

GROUP

SYSTEM

MANUAL

START / OFF / RESTART / MESSAGE

DAT/TIM

INIT

CHANGE

MCU

COPY

START

STOP

ONE-OFF

EXT

AUTO

PRINT

NEXT

FUTURE

OTHER

SUN

PRINT

VALVES

(PER*)

NUTRIE.

INFLU.

ALARM

FIXED

CYCLIC

CONTACT

VALVES

TIME

VOLUME

GENERAL

OTHER

(PER*)

FLOW

INFLU.

EXT INJ

(PER*)

(PER*)

(PER*)

(PER*)

TYPE

OUTPUTS

ACTIVA.

PERIODS*

OTHER

STATION

INPUT

FLOW

GENERAL

CART

OTHER

CONTACT

CFG CNT

INFLU

CONFIG

CONFIG

STATION

GROUP

AUTO

TEST (E/S)

Kp / Ki

SUN

FROST

LINKS

HEATING

DRAIN

MAIN  MENU

PRINT

DRAIN

HISTORY

PRINT

RESET

GROUP

GROUP

STATION

HEATING

HISTORY

EXT INJ

HEATING

DRAIN

XXXXX    = chapter in this guide.

TEST

PARAM

START

VALVES

EC / PH

GENERAL

OPTIONS

CODE

INSTALL

INJ CTL

CFG CNT=config. multifunction contacts; HEATING=irrig. water pre-heater;  INJ CTL=direct injection based on station flow; DRAIN=drainage ; EXT=external contact; INIT=initialization ; EXT INJ=phyto. /proportional injection; SUN=solar radiation; PERIODS* or  (PER*)= visible if number of periods >1.

# System parameters

## GENERAL CONFIGURATION

SYSTEM

| System parameters |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Thursday 01 January 1998 : 08h34 : 17h29 |  |  |  |  |  |  |  |  |  |
| INSTALL |  | OPTIONS |  | TIM/DAT |  | CODE |  |  | QUIT |



Visible with ‘installation’ access code

This menu displays the system date as well as the corresponding sunrise and sunset times calculated from the geographical data entered.

SYSTEM  OPTIONS

| System Options |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M C U |  | COPY |  | PRINT |  |  |  |  | QUIT |



Visible if access level  > 3

As well as providing the setpoint copy and printout functions, this menu allows users with level 4 access (typically installers) to define specific controller options (shown overleaf).

### Controller options

SYSTEM  OPTIONS  M.C.U. (Installation menu - level 4)

| MCUFerti Parameters |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit number : 1 MCUFerti name : MCUFerti PIA Alarm : YES Fixed times mem. store : 60 min Acquisition cycle time : 10 min Load history to PC : YES Type of solarimeter : 5..25 mA |  |  |  |  |  |  |  |  |  |
|  |  |  |  | xxx |  | ACCEPT |  |  | QUIT |



Function key (F3)

| - Unit number ( 1  4 ) | The default unit number is 1. It remains unchanged irrespective of any modifications made as long as the unit is not networked. However, when several units are connected to a network (RS485), each one must have a unique ‘unit number’. |
| --- | --- |
| - MCUFerti name ( 0  10 characters ) | The MCUFerti name can have a maximum of 10 characters. The function key F3 provides sequential access to characters "A" to "Z", while numbers are entered from the numeric key pad. |
| - PIA Alarm ( YES / NO ) | Enables or disables the input/output board fault monitor alarm (defaults to 'YES'),. |
| - Fixed times mem. store ( 0  120 minutes ) | Fixes the duration for which the fixed time watering schedule is stored in memory following loss of power or any other spurious cause of alteration to the schedule. |
| - Acquisition cycle time ( 0 – 1440 min ) | ***special function*** |
| - Load history to PC ( Oui / Non ) | Updates the PC with the details of watering starts occurring during periods of controller-to-PC down time. |
| - Type of solarimeter ( 0..4mA/5..25mA ) | Type of solarimeter used in the installation. [Please consult your installer for further information]. |

### Copy  settings between Groups

SYSTEM  OPTIONS  COPY

| Copy group |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ec/pH setpts : YES Source group : 1 Valve setpts : YES Start setpts : YES Dest. group : 1 Gen. setpts : YES |  |  |  |  |  |  |  |  |  |
|  |  | YES/NO |  | COPY |  |  |  |  | QUIT |



Visible depending on level

Selecting  COPY   will copy the setpoint values of categories marked with a "YES" from the source group to the destination group.

### Printing main Group settings

See § 9 PRINT MENUS

### System date and time

SYSTEM  DAT/TIM

| Setting date and time |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Day : Thursday Date : 10/12/97 Time : 00h00 Latitude : 44.2 deg Longitude : -0.3 deg D_GMT : 1 h Enable daylight saving : Yes Date summertime : 01/04 +1h Date winter time : 01/11 -1h Changeover time : 3 h |  |  |  |  |  |  |  |  |  |
|  |  |  |  | DAY |  | ACCEPT |  |  | QUIT |



Appears only when the cursor is on DAY

| - Latitude ( -90.0 °  +90.0 ° ) | Sets the latitude (in degrees) of the site as measured from the equator. |
| --- | --- |
| - Longitude ( -180.0 °  +180.0 ° ) | Value (in degrees) of the longitude of the site as measured from the Greenwich meridian. |
| - D_GMT (-12/+12 hr) | Enter the location time difference from GMT |
| - Enable daylight saving (Yes / No) | Enables daylight saving time (DST) |
| - Date summertime (DD/MM/YY) | Specify the date for changeover to summer time. |
| - Date winter time (DD/MM/YY) | Specify the date for changeover to winter time. |
| - Changeover time ([ 00h00 – 23h59 ]) | Specify the time of day at which DST occurs. |

### System access codes

SYSTEM  CODE

| Access Codes : level 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enter your code : ***** |  |  |  |  |  |  |  |  |  |
| INIT |  | CHANGE |  |  |  | ACCEPT |  |  | QUIT |

	

Visible if level > 4

visible if level  3

Access codes provide a degree of security in the management of your system. They correspond to levels of access, from low to high, allowing increasingly sensitive operational tasks to be carried out. Initially, all codes have standard values corresponding to the unit's 6 levels of security. Once a valid code has been entered the level is shown at the top of the display.

Settings at level 4 (and above) can seriously disrupt the unit's operation: you are advised to avoid operating at this level without first consulting your installer. The  INIT   option key allows ALL MCUFerti parameters to be initialized. Again, do not use this option without first contacting your installer.

At level three and above the codes may be changed, as explained on the following page.

SYSTEM  CODE  CHANGE

| Code change |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level 3 display Level 1 code : 1111 Ref. level : 1 Level 2 code : 2222 Time-out(min): 15 Level 3 code : 3333 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ACCEPT |  |  | QUIT |

| Code change |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level  4 display Level 4 code : 01209 Ref. level : 1 Time-out(min): 15 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ACCEPT |  |  | QUIT |

| - Level "x" code ( 0  99999 ) | Modify the code allocated to a given level. The codes for levels 1, 2 and 3 can only be altered at level 3. Higher level codes can only be changed at the corresponding level. |
| --- | --- |
| - Ref. level ( 1  Current level ) | Reference level to which the system automatically returns if no keypad activity is detected during the time-out period (see below). |
| - Time-out (min) ( 0  60 ) | Sets the maximum period of keyboard inactivity after which the system returns to the reference level (see above). |

# SETTING UP GROUPS

## INTRODUCTION

Consider the following definition.

A group includes one or more irrigation valves for which the answers to the following questions are identical:

- which stock solution to use?

- how much and where to deliver the final feed solution?

- when to start watering?

Configuring a group, therefore, involves programming parameters for: the station that controls irrigation, the type of stock solution, the irrigation valves delivering the feed water, and the watering start mode conditions. In addition, a series of general settings exist to tailor operation of the group to specific requirements or features of the installation (see page 29  for the details).

IMPORTANT

Clearly, once a group has been set up for watering it should not require systematic modification. If this is not the case the parameters are inadequately defined (in terms of the answers to the questions above). The correct remedy for such a situation involves setting up additional groups to meet the exact operational requirements, rather than continually reprogramming the parameters of an existing group.

To begin with select  GROUP  from the main menu display.

GROUP

| Choice of group |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Name Active Station 1 NONE NO 1 2 NONE NO 1 3 NONE NO 1 4 NONE NO 1 |  |  |  |  |  |  |  |  |  |
| Ec / pH |  | VALVES |  | STARTS |  | GENERAL |  |  | QUIT |

The direction arrows allow you to select a group to work with. Otherwise, press the lower    key to enter the number of the group directly.

## EC / PH SETPOINTS

GROUP  Ec/pH

| Ec/pH setpoints group 1 |  |  |  |  |  |  |  | 00:00:00 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Period : 1 pH setpoint : 5.5 pH Ec setpoint : 2.5 Ec Inlet Ec setpoint : 0.0 Ec Inlet Ec offset : NO Influence on inlet Ec : NO |  |  |  |  |  |  |  |  |  |  |
| NUTRIE. |  | ALARMS |  | INFLUE. |  | ACCEPT |  |  | QUIT |  |

		

Alarm tolerances

Influences on the Ec setpoint

Nutrient dosage

Visible if period number  > 1

Visible for inlet Ec influence option

Visible for Ec pre-regulation option with 3-way valve (= recirculation)

| - pH setpoint ( 1.0  9.0 pH ) | Setpoint value the system uses to regulate the required degree of water acidity. At the extremes, settings of 1.0 and 9.0 inhibit alkali and acid injection, respectively. |
| --- | --- |
| - Ec setpoint ( 0.0  10.0 mS/cm ) | Setpoint value the system uses to regulate the feed water nutrient concentration. One Ec unit corresponds to 1 mS/cm. A value of 0.0 inhibits both the nutrient injection mechanism and the Ec tolerance alarm. (see 'Ec / pH alarm'). |
| - Inlet Ec setpoint ( 0.0  Ec setp. mS/cm ) | Setpoint value the system uses to regulate nutrient concentration at the mixing tank inlet. A 3-way valve upstream of the tank mixes clean water with recirculated water until the setpoint value is obtained. (Pre-regulation using a 3-way valve = recycling). |
| - Inlet Ec offset ( YES / NO ) | OPTION: pre-regulation with 3-way valve = recycling. A 'YES' entry causes the Ec setpoint at the inlet to change to an offset value calculated as follows: Inlet Ec offset = Ec setpoint - Inlet Ec setpoint. A 'NO' entry leaves the setpoint unaffected. |
| Key | Change the current period. |

| - Inlet Ec influence ( YES / NO ) | OPTION Setting this influence to 'YES' means the Ec setpoint will be altered as follows: Ec setpoint = Ec setpoint + Inlet Ec measurement, a process that ensures a constant supply of nutrient to the mixing tank. However, to obtain the inlet Ec measurement the monitor sensor must be sited at the mixing tank inlet, which means the ‘Ec monitor difference’ must be set to 10 in order to avoid false alarms (see §3.4). A value of 'NO' influence leaves the Ec setpoint unchanged. |
| --- | --- |

### Hydroponic option

GROUP   EC/pH

| Ec/pH setpoints group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Period : 1 pH setpoint : 5.5 pH Ec setpoint : 2.5 Ec Stop Ec regulation : NON Stop pH regulation : NON |  |  |  |  |  |  |  |  |  |
| NUTRIENT |  | ALARM |  | INFLU. |  | MODIFY |  |  | QUIT |

| - Stop Ec regulation (Oui / Non) | Enter 'Yes' to suspend Ec regulation. In this case, the Ec setpoint is forced to '0', and the 'Ec tolerance', 'Max Ec tolerance' and 'Ec monitor difference' settings are forced to ‘8’, and the 'Ec alarm delay' to ‘180’ (see § 3.4). |
| --- | --- |
| - Stop pH regulation (Oui / Non) | Enter 'Yes' to suspend pH regulation. In this case, the pH setpoint becomes equal to ‘9,0’, or to ‘1,0’ in the case of base (as opposed to acid) regulation. In addition, the 'pH tolerance', 'Max pH tolerance' and ' pH monitor difference' settings are forced to ‘8’, and the 'Ec alarm delay' to ‘180’ (see §3.4). |

## NUTRIENT QUANTITIES

GROUP  Ec/pH  NUTRIE.

| Nutrient proportions....100% max |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nutrient A percentage : 0 % Nutrient B percentage : 0 % Nutrient C percentage : 0 % Nutrient D percentage : 0 % |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ACCEPT |  |  | QUIT |

| - Nutrient percentage ( 0  100% ) | Represents the relative quantities of nutrient added to the water at each injection. The sum total must not exceed 100%. Where the total is less than 100%, the MCU first calculates the injection time for the highest nutrient percentage. The remaining injection times are calculated in terms of their ratio to the highest. |
| --- | --- |

Example:

Let

A = 20 %, B = 50 %, C =10 %, and

the injection base time for B = 10 sec

Hence

The nutrient used to calculate the injection base time =  B.

The injection time calculated for A = 10 x (20/50)	= 4 sec, and

the injection time calculated for  C = 10 x (10/50)	= 2 sec.

- NOTE

For More/Less (+/-) stations the nutrient percentage parameters represent the activation (or not) of the nutrient lines. A value greater than '0' will open the line for the duration of watering irrespective of the actual percentage value.

## EC/PH ALARMS

GROUP  Ec/pH  ALARM

| Ec/pH alarms group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ec tolerance : 0.5 Ec Max. Ec tolerance : 1.5 Ec Ec monitor difference : 1.0 Ec Ec alarm delay : 60 sec pH tolerance : 0.5 pH Max. pH tolerance : 1.5 pH pH monitor difference : 1.0 pH pH alarm delay : 60 sec |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFY |  |  | QUIT |

| - Ec tolerance ( 0.0  10.0 mS/cm ) | Maximum permitted difference (+ or -) between the Ec measurement and the setpoint. If the tolerance is exceeded during watering the injection of nutrient stops. This causes the station to alarm if 'Pause on Ec/pH alarm' (see §3.10) has been set to 'YES', or watering to continue without nutrient if set to 'NO'. |
| --- | --- |
| - Max Ec tolerance ( 0.0  10.0 mS/cm ) | Maximum allowable positive (higher) difference between the Ec measurement and the setpoint. If this figure is exceeded the station automatically alarms. |
| - Ec monitor difference ( 0.0  10.0 mS/cm ) | Maximum difference between the main and monitor Ec sensor measurements. In the absence of a monitor sensor this value should be set to '10.0'. The 'Pause on Ec/pH alarm' setting operates as described above for Ec tolerance. |
| - Ec alarm delay ( 0  600 sec ) | Period before the alarm annunciates following detection of any of the above alarmable conditions (the condition must be continuous). |
| - pH tolerance ( 0.0  8.0 pH ) | Maximum difference (+ or -) between the main pH measurement and the setpoint. If the tolerance is exceeded during watering acid/base injection will stop. This causes the station to alarm if 'Pause on Ec/pH alarm' (see §3.10) has been set to 'YES', or watering to continue without nutrient if set to 'NO'. |
| - Max pH tolerance ( 0.0  8.0 pH ) | Maximum difference between the pH control setpoint and the measurement (higher for base injection, lower for acid injection). If this tolerance is exceeded the station automatically alarms. |
| - pH monitor difference ( 0.0  8.0 pH ) | Maximum difference between the main and monitor pH sensor measurements. In the absence of a monitor sensor set this parameter to 8.0 (the 'Pause on Ec/pH alarm' setting operates as described above for pH tolerance). |
| - pH alarm delay ( 0  600 sec ) | Period before the alarm annunciates following detection of any of the above alarmable conditions (the condition must be continuous). |

## INFLUENCES ON EC

GROUP  Ec/pH  INFLU.

| Modif. Ec by solar energy Group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERIOD : 1 Energy influence range max. : 1000 W/m2 Energy influence range min. : 500 W/m2 Max. Ec modification : 0.0 mS/cm |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | ACCEPT |  |  | QUIT |



| Energy influence range max. Visible if period number > 1 (Energy influence range min.  1000 W/m2 ) | Upper solar energy range limite above which no further reduction of the Ec setpoint takes place (that is, the reduction equals the 'Max. Ec modification' setting described below). |
| --- | --- |
| - Energy influence range min. ( 0 W/m2  Energy influence range max.) | Lower solar energy range limit below which no further reduction of the Ec setpoint takes place (that is, the reduction is the minimum allowed, or zero). |
| - Max. Ec modification ( 0.0  10.0 mS/cm ) | Maximum permitted reduction of the Ec setpoint due to solar radiation. The reduction is proportional to the energy measurement over the above defined range. The setting is programmed for a given period, which can be changed using the PERIOD key (F1). |

## VALVE PARAMETERS

GROUP  VALVE

| Valve setpoints group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Valve control mode : TIME No. of simultaneous valves : 1 |  |  |  |  |  |  |  |  |  |
| TIME |  | VOLUME |  | VALVES |  | GENERAL |  |  | QUIT |

			

Volume based control

Selecting valves

Time based control

General setpoints

This menu provides access to irrigation valve setpoints for valves in a specific group. The display also gives a reminder of the valve control mode chosen for the group (TIMED or VOLUME), as well as the number of valves programmed to operate simultaneously.

### Time based valve control

GROUP  VALVE  TIME

| Valve setpoints group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible if period number > 1 PERIOD : 1 Irrigation time : 00:00:00 Wetting time : 0 sec Max. pre-mix time : 0 sec Level reduction time : 0 sec Filling time : 0 sec Sensor rinse time : 0 sec Rinse time : 0 sec Wetting valve : 0 Rinsing valve : 0 |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | ACCEPT |  |  | QUIT |



| - Irrigation time ( 00:00:00  07:59:59 ) | Basic watering time allocated to each valve or block of valves in a group. This stage corresponds to the delivery of nutrient solution through the valves to the irrigation system. The setting can be programmed per period, which can be changed using the PERIOD key (F1). |
| --- | --- |
| - Wetting time ( 0  1800 sec ) | Wetting time for each valve or block of valves in a group. This stage corresponds to the circulation of clean water through the valve zone. Enter zero to inhibit this stage. |
| - Max. pre-mix time ( 0  600 sec ) | Maximum time allowed for the mixing tank to deliver the correct values of Ec and pH; that is, for the measurements to come within tolerance (see Ec/pH alarms). With the valve zone closed, nutrient in injected and the solution agitated until the mixture is correct, at which point the pre-mixing stage terminates. Enter zero to inhibit this stage. Premixing starts with a 10 second period during which the pump operate in order to rinse the pipework with tank water. No nutrient is injected during this period. |
| - Level reduction time ( 0  180 sec ) | Together with the tank low level sender signal this setting is used to calculate the optimum moment at which to close the inlet, in order to ensure the mixing tank is (virtually) empty at the end of watering. For example, if no level sender signal is received during the set period, watering ends normally and the system increases the level reduction time by 5s (up to 180s maximum). After several cycles of irrigation the system will have calculated the optimum level reduction time. A setting of zero will inhibit this function. |
| - Filling time ( 0  180 sec ) | Determines the time allowed for the mixing tank to fill with water. Enter zero to inhibit this stage. |
| - Sensor rinse time ( 0  180 sec ) | Period of mixing pump operation occurring before the end of the filling stage. If there is no filling stage this setting has no effect. |
| - Rinse time ( 0  1800 sec ) | Duration of the rinsing stage that follows watering, during which clean water is flushed through the valve zone to rinse each valve or block in a group. Enter zero to inhibit this stage. |
| - Wetting valve | Select a specific valve to be opened for wetting. Enter zero to inhibit the effect of this setting. (See the example below). |
| - Rinsing valve | Select a specific valve to be opened for rinsing. Enter zero to inhibit the effect of this setting. Example: If the parameter value = 16, valve n°16 will open during the rinsing stage. If the parameter value = 0, all valves in the group will open for the programmed rinsing time. |

### Volume based valve control

GROUP  VALVE  VOLUME

| Valve setpoints group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible if period number > 1 PERIOD : 1 Irrigation volume : 0 l Wetting volume : 0 l Max. pre-mix time : 0 sec Level reduction volume : 0 l Filling time : 0 sec Sensor rinse time : 0 sec Rinsing volume : 0 l Wetting valve : 0 Rinsing valve : 0 |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | ACCEPT |  |  | QUIT |



| - Irrigation volume ( 0  30000 l ) | Basic watering volume for each valve or block of valves in a group. This stage corresponds to nutrient solution delivery through the valves to the irrigation system. The setting can be programmed per period, which can be changed using the PERIOD key (F1). |
| --- | --- |
| - Wetting volume ( 0  10000 l ) | Wetting volume for each valve or block of valves in a group. This stage corresponds to the circulation of clean water through the valve zone. Enter zero to inhibit this stage. |
| - Max. pre-mix time ( 0  600 sec ) | Maximum time the system allows for the mixing tank to deliver the correct values of Ec and pH; that is, for the measurements to come within tolerance (see Ec/pH alarms). With the valve zone closed, nutrient is injected and the solution agitated until the mixture is correct, at which point the pre-mixing stage terminates. Enter zero to inhibit this stage. |
| - Level reduction volume ( 0  1000 l ) | Together with the low level sender signal the controller uses this setting to calculate the optimum moment at which to close the inlet to ensure the mixing tank is (virtually) empty at the end of watering. For example, if no level sender signal is received, watering ends normally but the system increases the level reduction volume by 5L (up to 1000L maximum). In this way, after several irrigation cycles the system will have calculated the optimum value. Enter zero to inhibit this function. |
| - Filling time ( 0  180 sec ) | Time allowed for the mixing tank to fill. Enter zero to inhibit this stage. |
| - Sensor rinse time ( 0  180 sec ) | Period of mixing pump operation occuring before the end of the filling stage. If there is no filling stage this setting has no effect. |
| - Rinsing volume ( 0  1000 l ) | Volume of clean water flushed through the valve zone following irrigation in order to rinse each valve or block of valves in a group. Enter zero to inhibit this stage. |
| - Wetting valve | Select a specific valve to be opened for wetting. Enter zero to inhibit the effect of this setting. |
| - Rinsing valve | Select a specific valve to be opened for rinsing. Enter zero to inhibit the effect of this setting. (See the example in section 3.6.1). |

### Selecting valves to a group

GROUP  VALVE  VALVES

| Select valves group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Val % Time Litres cc/dp l/m2 1 100 00:00:00 0 0 0.0 |  |  |  |  |  |  |  |  |  |
| PERIOD |  | FLOW |  | INFLU |  | ACCEPT |  |  | QUIT |

		 

Visible if period  number > 1.

No display of the current period

Visible if option Ec influence per valve

| - Val ( 0  No. of valves per group ) | Add a valve to the group. Valves will operate sequentially in list order. Enter '0' to remove a valve from the list. |
| --- | --- |
| - % column ( 0  900 % ) | Percentage applied to the basic watering time or volume to determine the actual time or volume for the valve. The limit for TIME-based control is fixed at 23:59:59. |

The columns to the right of the percentage column contain either the valve time or volume. The last two columns show the estimated equivalent volumes per drip emitter (in cm3) and per square meter (in litres), which are only reliable if you have configured the correct physical parameters for each valve.

- NOTE

The percentage column may be used to compensate for small differences in flow between valves. However, do NOT use a value of '0%' to disable valves except as a temporary fix (for example, when a valve breaks) since, amongst other things it will lead to statistical inaccuracies in the readings.

### Option: ‘Influence Ec per valve’

GROUP  VALVE  INFLU

| Selection valves group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible if n° periods > 1 Val % Ec Period : 1 1 100 2.5 6 80 2.0 12 90 2.2 14 110 2.7 |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | MODIFY |  |  | QUIT |

This option allows you to set different Ec setpoints for each valve in the group. For example, you may wish to use this function to vary the nutrient solution for plants at different stages of development.

| Val | Displays the valve numbers listed in the ‘Selecting valves to a group’ menu. |
| --- | --- |
| - % column | Set the Ec setpoint for the valve in terms of the percentage of the basic Ec setpoint. The default value is '100%'. |
| Ec | Displays the resultant Ec setpoint calculated for each valve. |

### Valve physical parameters

GROUP  VALVE  VALVES  FLOW

| Valve parameters |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Val l/h/Dp No.Dp m2 St Drn Drn Dp 1 0.0 0 0 1 0 0 2 0.0 0 0 1 0 0 3 0.0 0 0 1 0 0 4 0.0 0 0 1 0 0 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | ACCEPT |

| - l/h/Dp ( 0.0  999.0 l/h) | Nominal flow rate of the irrigation drip emitters supplied by the valve. |
| --- | --- |
| - No. dp ( 0  120000 ) | Number of drip emitters supplied by the valve. |
| - m2 ( 0  30000 m2) | Surface area of the crop supplied by the valve. |
| - St ( 1  N° of stations in the installation) | Number of the station to which the valve belongs. |
| - Drn ( 0  N° of drainage lines in the installation) | Number of the drainage line associated with the valve for measurement purposes. The entry is ignored if the installation does not incorporate drainage measurement. Otherwise, a value of '0' forces the system to disregard drained water measurements. |
| - Drn Dp ( 0  120000 ) | Number of drip emitters supplying the area over the collecting tray feeding the drainage line (see chapter 7). The entry is ignored if the installation does not incorporate drainage measurement. |

### General valve setpoints

GROUP  VALVE  GENERAL

| Valve setpoints group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible with 3-way valve pre-regulated Ec option Valve control mode : TIME No. simultaneous valves : 1 Valve interval : 0 sec Flow alarm delay : 30 sec Flow tolerance : 30 % Flow alarm : NO Minimum flow : 0 l/h Tank empty ("aspi") input : 0 Min. clean water time : 0 sec |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ACCEPT |  |  | QUIT |

| - Valve control mode ( TIME/VOLUME ) | Parameter instructing the system to use either the time or volume based setpoints. |
| --- | --- |
| - No. simultaneous valves ( 1  4 ) | Defines the number of valves controlled simultaneously during watering: the valves in the group list are divided into equal blocks (although the final block may contain less valves). Note: a separately available option allows simultaneous control of up to 8 valves. |
| - Valve interval ( 0  300 sec ) | Determines the opening and closing interval between successive blocks of valves: the system suspends nutrient regulation during this period since watering is paused. |
| - Flow alarm delay ( 0  180 sec ) | Period before the alarm annunciates following the detection of a flow rate measurement error (the alarmable condition must be continuous). |
| - Flow tolerance ( 0  100 % ) | Defines the maximum permitted difference (+ve & –ve) between the measured and theoretical flow values, expressed as a percentage of the theoretical value (which depends on the valve physical parameters): valid flow rate measurements come within the range given by: range = theoretical flow  tolerance. |

| - Flow alarm ( YES  NO ) | Enables or disables the station alarm in response to a flow measurement error: 'YES' causes the station to stop and the alarm to annunciate, 'NO' activates the next block of valves in the sequence (if the sequence has terminated watering ends as soon as the alarm delay has elapsed). In the absence of a flow meter the flow tolerance must be set to  100 % to prevent unwanted alarms, since the measurement effectively remains at zero (diagrams 1 to 4 on the following page illustrate typical situations). |
| --- | --- |
| - Minimum flow ( 0  30,000 l/h ) | Defines the low flow threshold below which a measurement will generate an alarm if the condition persists beyond the delay time. |

Option: pre-regulation with 3-way valve

| - Tank empty aspi input ( 0  system aspi number ) | Number of the contact input allocated to the recirculating tank level sender: an empty tank (or 'aspi') signal will position the 3–way valve for clean water only. |
| --- | --- |
| - Min. clean water time ( 0  1900s ) | Minimum length of time the 3-way valve is positioned for clean water if a empty tank signal is detected. |

### Flow alarm diagrams

+ 30

– 30

relative (theoretical)

flow = (N dp) x (l/h/dp)

Tolerance

%

Time

flow

flow

flow

flow

Flow alarm = YES; flow within tolerance, therefore no alarm condition

1

+ 30

– 30

relative (theoretical)

flow = (N dp) x (l/h/dp)

Tolerance

%

time

flow

flow

flow

flow

Flow alarm = YES; flow out of tolerance; time-out exceeded;    alarm condition & station stops

Flow alarm = NO;   flow out of tolerance; time-out  exceeded;   next block of valves activated

 if no next block, irrigation stops

time-out

2

relative (theoretical)

flow = (N dp) x (l/h/dp)

Tolerance

%

time

flow

flow

flow

flow

Flow alarm = YES; flow out of tolerance; time-out exceeded,  therefore alarm condition & station stops

time-out

100

3

relative (theoretical)

flow = N dp x l/h/dp

Tolerance

%

Time

No flow meter present;  flow alarm = No;  tolerance must be set to 100 % to prevent alarms

flow

100

4

## START MODE SETTINGS

GROUP  START

| Start modes group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fixed time start mode : INACTIVE Cyclic start mode : INACTIVE Start contact mode : INACTIVE |  |  |  |  |  |  |  |  |  |
| FIXED |  | CYCLIC |  | CONTACT |  | OTHER |  |  | QUIT |

			

cyclical

contact (external)

at fixed times

other start modes

Start modes 

This menu provides access to all the settings concerned with the different group start modes, and displays the current state of each one (active or inactive).

### Watering at fixed times of day

GROUP  START  FIXED

| Fixed time starts group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time 1 : 00h00 Time 5 : Time 2 : Time 6 : Time 3 : Time 7 : Time 4 : Time 8 : |  |  |  |  |  |  |  |  |  |
| DEACTIV |  | ACTIVA. |  |  |  | ACCEPT |  |  | QUIT |

| - Time ( 00h00  23h59 ) | Eight possible fixed time starts are available for the current day. Pressing the ACTIVA. key allows a time to be entered, whereas the DEACTIV. key clears the entry. NOTE: start times 1 to 8 must be sequential. |
| --- | --- |

Once the details have been entered and accepted, the display shown below requires you to confirm activation or deactivation of the start mode (the display appears whichever start mode is being programmed).

| Fixed time starts group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activate fixed time starts ? |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | YES |  |  | NO |

### Cyclic watering

GROUP  START  CYCLIC

| Cyclic start group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible if period number > 1 PERIOD : 1 Cyclic start interval : 10 min Activate start mode : 07h00 End start mode : 20h00 |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | ACCEPT |  |  | QUIT |



| - Cyclic start interval ( 5  1440 min ) | Sets the duration of the interval between successive water starts: the setting is programmable per period (changed using the PERIOD key, F1), though this does not affect the times at which activation of the mode begins and ends. |
| --- | --- |
| - Activate start mode ( 00h00  23h59 ) | Time at which the group cyclic start mode activates. |
| - End start mode ( 00h00  23h59 ) | Time at which the cyclic start mode deactivates. |

### Contact started watering

Introduction

Contact inputs are reserved specifically for starting and stopping groups. The measurement range is 0-100%. The functions linked to these inputs depend on a programmable threshold that determines each contact's operating state (activated or deactivated).

There are  three categories of contact: '0-100%', 'Resistive', and 'Ext '.

- '0-100%' designates the station's contact inputs, up to a maximum of 4

- 'Resistive' refers to a separately designated station input

- 'Ext'  refers to external inputs (not specific to any station), the number and function of which varies depending on the installation. These contacts can be assigned to any group belonging to any station.

The menu to program the water 'Start' contact (number and activation threshold) is shown overleaf. The precise effect on watering of its status can be set in the general menu, section §3.10.4.

Finally, station and external contacts are enabled and configured through the installation menus in sections §10.7,  §10.8 and §10.9.

GROUP  START  CONTACT

| Contact start group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible if period number > 1 PERIOD : 1 Contact start interval : 10 min Contact number : 1 Activation threshold : 50 % Activate start mode : 07h00 Deactivate start mode : 20h00 |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | ACCEPT |  |  | QUIT |



| - Contact start interval ( 0  1440 min ) | Minimum interval the system imposes between successive contact water starts: it can be programmed per period, which can be changed with the PERIOD key (F1). |
| --- | --- |
| - Contact number ( 1  No. of station contacts, or 1  No. of external contacts) | Specifies either the number of the station or external contact input used to trigger watering (see also §3.10.4). |
| - Activation threshold (1 100%) | Sets the measurement threshold used to trigger the group: the origin of the measurement is fixed by the number of the contact. This setting can be programmed per period (changed using the PERIOD key, F1), though this does not affect the activation start and end times. |
| - Activate start mode ( 00h00  23h59 ) | Time at which contact started watering is activated for the group. |
| - Deactivate start mode ( 00h00  23h59 ) | Time at which contact started watering is deactivated for the group. |

## OTHER IRRIGATION START MODES

GROUP  START  OTHER

| Start modes group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solar energy start : INACTIVE Frost start : INACTIVE |  |  |  |  |  |  |  |  |  |
| SUN |  | FROST |  | DRAIN |  | LINKS |  |  | QUIT |

			

Solar radiation start mode

Start mode links

Visible if drainage option exists

This display provides access to the settings that configure the other available watering start modes, and also displays the currently active modes.

### Solar energy start mode

GROUP  START  OTHER  SUN

| Solar energy start group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible if no. of periods > 1 PERIOD : 1 Solar energy start interval : 30 min Min. intensity level : 20 W/m2 Energy start threshold : 400 Wh/m2 Activate start mode : 10h00 End start mode : 06h00 Daily carry over : NO Cumulative starts limit : 1 Accumulate when inactive : NO |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | ACCEPT |  |  | QUIT |



| - Solar energy start interval ( 0  1440 min ) | Minimum interval between successive watering starts triggered by measurement of solar radiation: the interval can be programmed per period. |
| --- | --- |
| - Min. intensity level ( 0  999 W/m2 ) | Starting point for the totalized (or accumulated) measurement of solar energy for the group. This parameter can be programmed per period (changed using the PERIOD key F1), which does not affect the start and end times or the last three settings at the bottom of the menu. |
| - Energy start threshold ( 0  9999 Wh/m2 ) | Measured amount of accumulated solar energy required to trigger a water start for the group. |
| - Activate start mode ( 00h00  23h59 ) | Time at which this start mode is activated for the group. |
| - End start mode ( 00h00  23h59 ) | Time at which this start mode is deactivated. |
| - Daily carry over ( Yes /No ) | Specifies whether or not the residual solar energy measurement from one day can be carried over and used as the starting point for the following day's total. |
| - Cumulative starts limit ( 1  3 ) | Upper limit on the cumulative solar energy measurement expressed as a number of permitted solar energy watering starts. This parameter functions by preventing watering starts that 'fail' (blocked by another mode, for example) from increasing the solar energy totalizer. |
| - Accumulate before mode active (Yes /No) | Enables solar energy accumulation to begin before the start of the activation period. The totalizer is activated at midnight. |

### Start mode links

GROUP  START  OTHER  LINKS

| Links configuration group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FIXED SUN CYC CONT FROST SUN – – – – - CYC – – – – - CONT – – – – - FROST - - - - - |  |  |  |  |  |  |  |  |  |
| ACTIVA. |  | DEACTI. |  |  |  | MODIFY |  |  | QUIT |

Operators with level 3 access and above can create links between different start modes by using the possibilities offered in the above table. Checking a link field means that a given start mode will delay the activation of another, the mode in the column having priority over the mode in the line, as shown by the arrow.

Overall, the effect is to reduce the watering interval of the target mode to its nominal value (or if solar energy start mode is targeted, to reduce the accumulated energy total).

## FROST CONTACT START MODE

The first group frost start will occur when one of the two specified external contacts operates. Thereafter, the group will continue to water cyclically until manually stopped.

GROUP  START   OTHER  FROST

| Setpoints frost contact group 1 |  |  |  |  |  |  |  | 10:12:25 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible if no. periods > 1 Period : 1 Contact start interval : 25 mn Activation threshold : 1.0 Contact number 1 : 1 Contact number 2 : 0 |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | MODIFY |  |  | QUIT |

| - Contact start interval ( 0 – 1440 min ) | Interval between successive frost contact water starts for the group: programmable per period. |
| --- | --- |
| - Activation threshold (-99,9 / 999,9 ) | Contact measurement threshold used to start the group for the first time: programmable per period. |
| - Number of contact 1 ( 1 – no. ext contact ) | Number of the external contact used to start the group for the first time. |
| - Number of contact 2 ( 1 – no. ext contact ) | If the number programmed here is not '0', the first frost start will take place if either contact 1 or contact 2 operates. |

Once the details have been entered and accepted, the display shown below requires you to confirm activation or deactivation of frost start mode.

|  |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activate frost starts ? |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | YES |  |  | NO |

### Stopping a frost contact started group

Once a frost contact group has started, watering occurs cyclically at intervals defined by the 'contact start interval' parameter. Thereafter, watering will stop only if:

- the group is manually stopped during watering, or

- the group is deactivated, or

- the frost contact start mode is deactivated.

If both group and frost contact start mode are activated,  watering will (re)start when either of the external contacts operates.

### Frost starting with 'Multifunction Contacts'

An Elotec temperature sensor can be connected to an MCU+ multifunction /multistation contact input (see §10.9) and be programmed to operate for frost start detection. The measurement appears directly in the screen containing the external contact readings.

Firstly, the chosen contact must be configured as type ‘NTCA’ in the multifunction contact menu (path = SYSTEM  INSTAL  OTHER  CFG CNT ).

Next, it must be made to operate when the temperature measurement falls below the programmed threshold. This requires contact operation to be inverted, which is done by setting the ‘Contact inversion’ setting to ‘YES’ in the installation menu (path = SYSTEM  INSTAL  OTHER  CFG CNT).

The group will then be frost started if the temperature of the sensor connected to the contact input drops below the ‘Operating threshold’ setting in the frost start menu.

- NOTE

Frost starting is handled in the same way as other types of group start; that is, watering occurs according to the priority and start times defined for each group.

## GENERAL SETTINGS

GROUP  GENERAL

| General setpoints group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PERIODS |  | OUTPUTS |  | ACTIVA. |  | OTHER |  |  | QUIT |

			

Activation of the  outputs (visible depending on access level)

Activation of a  group

Other setpoints

Visible with  period option

This menu provides access to all general settings applicable to a particular group.

### Periods (available as an option)

GROUP  GENERAL  PERIOD

| Periods group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of periods : 4 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | OK |

This option is only visible with level 3 access or above. The maximum number of periods provided depends on the number ordered by the customer.

GROUP  GENERAL  PERIODS

| Periods group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Period Time Sunrise Sunset Type 1 00h00 -514 -1054 FIXED 2 06h00 -154 -694 S_RISE 3 12h00 206 -334 S_SET 4 18h00 566 26 FIXED |  |  |  |  |  |  |  |  |  |
|  |  |  |  | TYPE |  | MODIFY |  |  | QUIT |

Periods can be defined (and modified) in terms of either fixed times of day, or in relation to sunrise and sunset. Begin by choosing a 'Type' column entry, then move left to enter values in the other columns, as appropriate. The start times of consecutively numbered periods must be sequential between 00:00 hrs and 23:59 hrs.

### Programming outputs

GROUP  GENERAL  OUTPUTS

| Outputs configuration group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INT WET PRE REG RED FIL RIN AUX1 * - - - - - - AUX2 - - - - - - - AUX3 - - - - - - - AUX4 - - - - - - - PMP1 * * - * - * * PMP2 - * * * * - * |  |  |  |  |  |  |  |  |  |
| ACTIVA. |  | DEACTI. |  | OTHER |  | MODIFY |  |  | QUIT |



Visible with optional extra auxiliaries (XA)

(see  below)

This menu is visible for access levels 3 and above. The table rows represent outputs (auxiliaries and pumps), and the columns the different stages of watering (listed below). The choice of assigning options to stages is determined by the kind of installation you have. In case of any doubt about which selections are appropriate you are strongly advised consult your installer.

| Start (INT) | = start up interval between pump 1 and pump 2. |
| --- | --- |
| Wetting (WET) | = clean water circulated through the irrigation valves. |
| Premixing (PRE) | = obtaining the required nutrient pre-mix in the tank. |
| Regulation (REG) | = distribution of regulated nutrient solution through the valve zone and irrigation system. |
| Reduction (RED) | = tank level reduction as watering ends. |
| Filling (FIL) | = filling the mixing tank with clean water. |
| Rinsing (RIN) | = clean water circulated through the irrigation valves. |

More outputs

| Outputs config. group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INT WET PRE REG RED FIL RIN XA 1 * - - - - - - XA 2 - - - - - - - XA x - - - - - - - XA10 - - - - - - - XA x * * - * - * * XA16 - * * * * - * |  |  |  |  |  |  |  |  |  |
| ACTIVA. |  | DEACTI. |  |  |  | MODIFY |  |  | QUIT |

### Activating a group

GROUP  GENERAL  ACTIVA.

| Activation group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sun Mon Tue Wed Thu Fri Sat _ _ _ _ _ _ _ Day interval : 1 days |  |  |  |  |  |  |  |  |  |
| ACTIVA. |  | DEACTI. |  |  |  | MODIFY |  |  | QUIT |

A group can either be activated on a regular day interval basis (how often) or on particular days of the week. Select by positioning the arrow on the left hand side of the display and entering the details.

| - Weekly calendar | Each box ticked activates the group on the corresponding day (accessible using the ACTIV. and DEACTI. options when the arrow is in the upper position). |
| --- | --- |
| - Day interval ( 1  9 days ) | Day interval period required, i.e. every 'n' days the group will be activated (enter a number with the display arrow in the lower position). |

Activation or deactivation of the group must be confirmed once the settings have been accepted.

| Activation group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Activate group 1 ? |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | YES |  |  | NO |

### More general settings

GROUP  GENERAL  OTHER

| General settings group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group name : UNNAMED Start interval : 0 min Pause on Ec/pH alarm : YES Irrigation priority : NO Type of start contact : DRY Contact operation : FLEETING Type of stop contact : 0-100% Contact number : 0 Delay before contact active : 5 sec Type of suspend contact : 0-100% Contact number : 0 Type of next valve contact : 0-100% Contact number : 0 Time before next valve : 5 sec Resistive contact inversion : NO Operating threshold : 50 % Regulate pH for premix : NO Duration of start stage : 0 sec |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFY |  |  | QUIT |

| - Group name ( 0  10 characters ) | Comprises a maximum of 10 characters: letters "A" to "Z" are entered using the F3 key, numbers are entered from the numeric keypad. |
| --- | --- |
| - Start interval ( 0  99 min ) | Minimum interval the system will impose between two successive group starts. |
| - Pause on Ec/pH alarm ( Yes /No ) | Specifies whether an Ec/pH alarm should also cause operation of the station to be suspended: if 'NO' is selected, watering continues in spite of the alarm, but without nutrient. |
| - Irrigation priority ( Yes /No ) | A 'Yes' entry gives the group priority over any irrigation in progress, which will be suspended. However, it cannot interrupt another group having priority on the same station. Entering 'NO' means the group can normally only water if a start mode is valid and the station is free. |
| - Type of start contact (0-100% / Resistive / EXT ) | Type of contact used to start the group (refer to section §3.7.3). |
| - Contact function ( FLEETING/HELD ) | 'FLEETING' (= non-locking control action) allows the group to start as soon as the contact signal is detected. Watering then continues as normal. 'HELD' (= locking control action) starts the group but only allows watering to continue while the signal is present; that is, if the contact changes state the process is stopped. Do not choose the HELD function for a 'STORED' contact (refer to section §10.8). |
| - Type of 'stop' contact (0-100% / Resistive / EXT ) | Type of contact used to stop the group and pass to the (tank level) 'Reduction' stage. |
| - Contact number (see §3.7.3) | Number of the contact input used to stop the group. Enter zero to cancel this setting. |
| - Delay before contact active ( 0 – 1440 min ) | Duration of the period following the group start during which the stop contact is disabled (i.e. cannot stop the group). |
| - Type of 'suspend' contact (0-100% / Resistive / EXT ) | Type of contact used to suspend the group, which will restart once the contact signal is no longer present. |
| - Contact number (see § 3.7.3) | Number of the contact input used to suspend the group. Enter zero to cancel this setting. |
| - Type of 'next valve' contact (0-100% / Resistive / EXT ) | Type of contact used to trigger the 'next valve in group sequence' command. |
| - Contact number (see § 3.7.3) | Number of the contact input used to open the next valve. |
| - Resistive contact inversion ( Yes /No ), and - Activation threshold (0-100%) | These two settings together configure the station's resistive contact input. If inversion is disabled ('NO'), the input signal is treated normally (that is, 'high=on low=off' above and below the threshold, respectively) If contact inversion is enabled ('YES') the high-low sense is reversed, and the contact operates when the input signal falls below the start threshold. |
| - Regulate pH for premix (Yes /No) | Specifies that pH regulation should take place during the pre-mixing stage. |
| - Start time ( 0  1440 sec ) | Determines the actual duration of the 'start' stage. Consult your installer for further information. |

## PROPORTIONAL INJECTION FUNCTION

NOTE

The function must be ordered as an added option for each MCU+.

### Principle of operation

Proportional injection – also called external injection – involves introducing a given solution at the station outlet in amounts proportional to the irrigation flow rate.

It is advisable to program a group (without Ec and pH control) reserved specifically for the use of this function.

Time-controlled injection valve

Station

Flowmeter

Solution

### Settings menus

External injections are programmed through the following 3 menus.

GROUP  GENERAL  OTHER  EXT INJ

| Setpoints external injections Group 1 |  |  |  |  |  |  |  | 10 :12 :15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Enable external injection : YES Theoretical flowrate : 5 l/h Irr. volume per litre injected : 500 l Minimum injection time : 3 Number of attempts before alarm : 4 Output number to activate : |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | O K |  |  | QUIT |

| - Enable external injection (Yes /No) | Enter 'Yes' to enable the function. |
| --- | --- |
| - Theoretical flow rate (1 - 30,000 l/h) | Flow rate of the solution to be injected through the valve: the value entered depends on how the function is used. |

| - Irrigation volume per litre injected (1 – 30,000 l) | Determines the dilution. The required figure is calculated as follows: Example: |
| --- | --- |
|  | If the objective is to inject 1.5 litres of plant protection solution for every 300 litres of irrigation water, the value of the setting is 200 (or 300/1.5) litres. |
| - Minimum injection time (30 – 500 x10ms) | Minimum operating time for the injection valve, in the range 30 to 500 hundredths of a second (300ms to 5sec). If the calculated time is less than this figure, the injection is cancelled and the value held in memory as a carryover for the next injection. |
| - Number of attempts before alarm (0 – 720) | The number of 5-second calculation intervals allowed before an alarm is activated. [Successive calculations producing > 5 second injection times = injection valve permanently open & objective not being attained]. Enter zero to disable the alarm. |
| - Output number to activate | Number of the MCU+ output used to control the injection valve (must be a valve control output). |

### External injection readings

READOUT  STATION  NEXT  EXT INJ

| External injection readout station 1 |  |  |  |  |  |  |  |  |  |  |  | 10 :12 :15 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flow rate : 10000 l/h Volume : 750.0 l Flow injected : 500 l/h Volume : 3.7 l |  |  |  |  |  |  | Valve : 1 2 Injection time : 277 * 10ms Total : 5 sec |  |  |  |  |  |  |  |  |
| HISTORY |  |  |  |  |  |  |  |  |  | O K |  |  |  |  | QUIT |

| Flow rate | = irrigation flow rate of the station |
| --- | --- |
| Volume | = volume irrigated so far |
| Flow injected | = calculated injection flow rate, representing a fractionated theoretical flow rate |
| Volume | = volume injected during irrigation |
| Valve | = number(s) of the valve(s) currently watering |
| Injection time | = injection time calculated every 5 seconds |
| Total | = total injection time accumulated over the watering period. |

At the end of watering all data is saved in a history file before being reset to zero in the display.

### History

READOUT  STATION  EXT  EXT INJ  HISTORY

| History external injections: 3/7 |  |  |  |  |  |  |  |  |  | 10:12:15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DD MM Time Station Group Vol Vol inj 10 09 09:30 n e 800.0 4.0 10 09 08:00 1 1 980.0 2.0 09 09 17:45 1 2 1500.0 3.5 09 09 16:00 2 3 1600.0 8.2 |  |  |  |  |  |  |  |  |  |  |
| INIT |  | PRINT |  |  |  |  |  |  | QUIT |  |

This table contains data for the last 10 start events of the groups programmed for proportional injection.

| DD MM Time | = date and time of irrigation. |
| --- | --- |
| Station | = number of the station assigned to the group that was started. |
| Group | = group that was started. |
| Vol. | = total volume of water supplied during irrigation. |
| Vol. inj | = total volume injected during irrigation. |
| INIT | = clears the table of data. |
| IMPRES | = prints out the data in a similarly formatted table. |

## DIRECT INJECTION WITH FLOW VARIATION

### Principle of operation

Nutrient injection volumes can altered to take account of large variations in station flow rate. The technique uses a set of parallel Venturi lines for each nutrient. The lines, numbered from 1 to 6, open and close in combination in response to the measured flow.

Each set of lines must identical so that, for each nutrient, similarly numbered lines must have the same hydraulic capacity (A1 = B1 = C1…, A2 = B2 = C2 …, A3 = B3 = C3….  and so on).

Nutrient A

Nutrient B

Flowmeter

A1  A2  A3  A4  A5  A6     B1  B2  B3  B4  B5  B6

For each nutrient, the relationship between the number of solenoid valves open and the measured station flow rate at any given moment may be illustrated graphically, as shown below.

To obtain such a control curve the installer must know the following:

- the figure for nutrient dilution and the corresponding Ec value (parameters that depend on the nutrient as supplied by the manufacturer), and

- the flow rate of each injection valve (given by the station's visual flow meter readings).

With this information, the flow rates D1 to D6, (which depend on the dilution objective for the nutrient) can be calculated and entered in the installer's menu shown further on.

Nutrient solenoid valves selected to open

Station flow

A1+A2+A3+ A4+A5+A6

A1

A1+A2

A1+A2+

A3

A1+A2+

A3+ A4

A1+A2+A3+ A4+A5

D1

D2

D3

D4

D5

D6

### Calculating the flow rates for the control graph

Firstly, for an Ec setpoint corresponding to the dilution Ec of the nutrient (quoted by the manufacturer), the flow rates are derived from the following formula:

Di =(Nutriant flow A1+...+ Nutriant flow Ai)Nutrient dilution (in 000)×1000

(where i = 1…6, and nutrient dilution is given in ‰)

If we wish to operate around the dilution Ec value, and taking Ec to be proportional to dilution, then for a given value of dilution, dilution Ec and a given Ec setpoint, the calculated flow rates are:

Di = 1000 * (Nutriant flow A1+...+ Nutriant flow Ai)Nutrient dilution (in 000)×Dilution EcEc Setpoint

However, the mother solutions in tanks A, B, …etc, made up by the grower will have a higher concentration and, therefore, a higher Ec than the manufacturer's dilution Ec figure (quoted for a dilution of 1‰,   1 kg solid nutrient dissolved in 1000 liters of water). Moreover, growers are more likely to work with ‘real’ dilutions expressed in percentages.

Solid nutrient with an Ec quoted for a dilution of 1‰

Nutrient A

'Real' dilution of solid nutrient in tank 'A', expressed in %

Flowmeter

In order to calculate the flow rates for the graph using the 'real' nutrient dilution figures we use the following relationship, where:

Nutrient dilution (in ‰)  =100Real dilution of the solid nutrient (in % )

The final formula for obtaining the flow rates therefore becomes:

Di =10 ×(Nutrient flow A1+...+ nutrient flow  Ai)  ×Real dilution (in %)×dilution EcEc setpoint

Conclusion

To optimize Ec regulation taking account of variations in station flow rate requires the following data: the flow rate of each injection valve; the figures for the real dilutions of the solutions in the tanks; the dilution Ec quoted by the manufacturer for each nutrient. Then, using these parameters the controller calculates the best injection to apply based on the programmed Ec setpoint and the measured station flow rate.

### Programming dilution and flow parameters

Nutrient dilution parameters (grower's menu)

GROUP  EC/PH

| Setpoints Ec / pH group 1 |  |  |  |  |  |  |  | 10:12:15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pH setpoint : 9.0 Ec setpoint : 2.6 Nutrient dilution : 1.0 % Dilution Ec : 1.3 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MOFIFY |  |  | QUIT |

| - Nutrient dilution ( 0 – 100 ) | Number of kilograms of solid nutrient dissolved in 100 liters of water, expressed as a percentage (= 'real' dilution). |
| --- | --- |
| - Dilution Ec ( 0 – 10 ) | Ec corresponding to a dilution of 1‰ (1kg solid nutrient dissolved in 1000 liters of water). |

Nutrient line flow parameters (installer's menu)

SYSTEM  INSTAL  GROUP  PARAM  INJ CTL

| Flow influence |  |  |  |  |  |  |  | 10:12:15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nutrient A0 : Flow rate : 0 l/h Nutrient A1 : Flow rate : 50 l/h Nutrient A2 : Flow rate : 150 l/h Nutrient A3 : Flow rate : 250 l/h Nutrient A4 : Flow rate : 400 l/h Nutrient A5 : Flow rate : 490 l/h Nutrient A6 : Flow rate : 700 l/h |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MOFIFY |  |  | QUIT |

| - Nutrient Ai : Flow rate ( 0 – 30,000 l/h) | Cumulative flow rate figures for the injection lines of a given nutrient. |
| --- | --- |

Excepting Venturi line A0 (which should be set to '0'), the flow rate for Venturi line 'i' must be the sum of the flow rates for lines A1 + A2 +… Ai. In other words, entries for nutrient lines 1 to 6 must progressively increase in value. If this is not the case an ERROR message is displayed and the entries are ignored.

### Programming station flow variation tolerances

SYSTEM  INSTALL  GROUP  PARAM

| Paramètres groupe 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station : 1 Kp Ec : 1.0 KpR Ec : 0.5 Ki Ec : 1.0 Kd Ec : 0.5 Max injection time Ec : 500 x10ms Initial injection time Ec : 0 x10ms Kp Ecp : 0.8 Kd Ecp : 0.5 Kp pH : 0.3 KpR pH : 0.2 Ki pH : 1.0 Kd pH : 0.5 Max injection time PH : 500 x10ms Initial injection time pH : 0 x10ms Ec/pH premix offset : 0 sec Ec pH maintain time : 0 sec Flow variation tolerance : 30 % Flow influence delay : 30 sec Min flow for influence : 100 l/h Type of flow influence : 1 Save initial injection time : NO Modif. Ec pH if station working : NO |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFIER |  |  | QUITTER |

| - Flow variation tolerance ( 0 – 100 %, or 0 – 30,000 l/h ) | Expresses either a percentage tolerance or an upper limit on the variation in flow: this depends on the 'Type of flow influence' setting (see overleaf). |
| --- | --- |
| - Flow influence delay ( 0 – 300 sec ) | Delays the flow influence calculation following a changeover to the next valve or group of valves: the deferred calculation takes into account the theoretical flow rate at the time of changeover. |
| - Min flow for influence ( 0 – 100 %, ou 0 – 30000 l/h ) | Expresses either a percentage tolerance or a lower limit on the variation in flow: this depends on the 'Type of flow influence' setting (see overleaf). |
| - Type of flow influence ( 0,1,2 ) | Type ‘0’ : 'Flow variation tolerance (0 – 100 %) 'Min flow for influence' (0 – 30000 l/h). "Flow variation tolerance" expresses a tolerance (+/-), and "Min flow for influence" sets the limit on flow variation. In other words, if the calculated flow variation based on tolerance (= tolerance x station flow) is less than the limit, then the "Min flow for influence" setting applies. Type ‘1’ : 'Flow variation tolerance' (0 – 100%) 'Min flow for influence' (0 – 100%). "Flow variation tolerance" is a tolerance applying to increases in flow rate and "Min flow for influence" is a tolerance that applies to decreases in flow. Type ‘2’ : 'Flow variation tolerance' (0 – 30000 l/h) 'Min flow for influence' (0 – 30000 l/h). 'Flow variation tolerance' limits the variation for increases in flow rate, and 'Min flow for influence' limits the variation for decreases in flow. |

- NOTE

This setting therefore determines when the direct injection with flow variation function is activated. In fact, it applies only for out-of-tolerance variations, as determined by this setting. Conversely, in-tolerance variations in station flow have no effect on injections which, therefore, remain under classic PID control.

| - Save initial injection time ( Yes /No ) | If this parameter is set to ‘No’, the initial injection time (used as the basis for calculation) is not updated when watering ends. |
| --- | --- |

## VALVE IN CART MODE FUNCTION

### Introduction

With this function, the MCU control output of any group valve can be programmed to operate for either irrigation valve or watering cart (= watering boom)1 control.

- NOTES

(i) Cart control is limited to three commands: START, RESTART and STOP, the actual process of watering being controlled independently by the cart system itself.

(ii) The function is enabled only if TIME-BASED valve control is programmed for the group and if an appropriate IRRIGATION TIME is set through the GROUP  VALVE  TIME menu.

The sequence of events in cart mode is as follows:

- the group and the station to which it belongs are started,

- the valve’s control output operates for the period of time required to ensure correct starting of the cart (depending on the system), after which it turns off,

- the cart continues watering until it reaches its end position where it operates an external contact; when the contact is detected the group reactivates the output to restart the cart as before,

- the restarting process repeats for a predetermined (programmable) number of times, after which the final contact signal either passes control to the next valve in the sequence or causes the station to stop.

### Configuring the carts

SYSTEM  INSTALL  STATION  GENERAL  CART

| Configuration of carts |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 :12 :15 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cart 1 2 3 4 |  |  |  | Contact 3 1 4 0 |  |  |  |  |  |  |  | Time 00 :00 :30 00 :01 :00 00 :02 :00 00 :00 :20 |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | O K |  |  |  |  |  | QUIT |

| - Cart | Number of the cart. |
| --- | --- |
| - Contact | Input number of the external contact used to (re-)start the cart. If ‘0’ is entered, the valve concerned will function normally for a period equal to the cart activation time. |
| - Time | Duration of output activation necessary to ensure the cart is properly started (a function of the cart system mechanism). |

### Selecting the group valves

GROUP  VALVE  VALVE

| Configuration of carts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 :12 :15 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Val. 10 3 6 8 | % 100 2 1 3 |  |  |  | Ca 0 1 3 2 |  |  |  | Time 00 :05 :00 00 :00 :30 00 :00 :20 00 :01 :00 |  |  |  |  | Liters 0 0 0 0 |  |  |  |  | Cc/dp 0 0 0 0 |  | L/m2 0 0 0 0 |
|  |  |  |  |  |  |  |  |  |  |  |  |  | O K |  |  |  |  |  |  | QUIT |  |

This menu is used to program the operating mode for each valve in a group: that is, either cart or normal valve mode. With the cart mode function the 'Ca' column is added to the existing menu to allow valves to be assigned a cart number, as follows:

- entering '0' in the ‘Ca’ column means the valve operates normally and the '%' column works as described in § 3.6.3,

- entering a positive number identifies the cart, and means the number of end-to-end travels it makes before stopping should be entered in the ‘%’ column.

Examples :

1. The number of simultaneous valves  = 1

Using the valve configurations given in the two previous menus, the irrigation sequence is: V10 (normal), V3 - V6 - V8 (cart), as shown in the events diagram below:

Valve 3

Valve 6

Valve 8

Group starts

Ext. contact 3

Ext. contact 4

Ext. contact 1

Group stops

Time

Cart 1

Cart 2

Cart 3

5 minutes (V10)

Valve10

(indeterminate)

2. The number of simultaneous valves  = 2

The irrigation sequence is now: V10&V3, V6&V8, as shown in the events diagram below:

Valve 3

Valve 6

Valve 8

Group starts

Ext. contact 3

Ext. contact 4

Ext. contact 1

Group stops

Time

Cart 1

Cart 2

Cart 3

5 minutes (V10)

Valve 10

(indeterminate)

- NOTE

At the present time watering the cart settings are not transmitted to the PC and so are not included in the set-ups (recipes) saved for re-use.

# MONITORING

## STATUS READINGS

This menu provides access to all monitoring functions, and has an initial display containing the current operating status of the station(s).

READOUT

| Current status |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station Status Group Start 1 Free 0 |  |  |  |  |  |  |  |  |  |
| EXT |  | STATION |  | GROUP |  | DRAIN |  |  | QUIT |

			

Group running totals & averages

Visible with drainage option

Station(s) readout

Visible with external contacts

Status can be:

| Free | = not operating, |
| --- | --- |
| Started | = currently operating (see details under "station readout"), |
| Alarm | = one or more alarm conditions have arisen (see alarm readout details), |
| Pause | = operation (irrigation) suspended. |

In addition, group number and start mode information is displayed for the Started, Alarm and Pause conditions.

Programmed start modes include:

| Fixed | = watering at fixed times of day, |
| --- | --- |
| Cyclic | = watering at fixed cycle intervals, |
| Contact | = watering in response to a contact input signal, |
| Light meter | = watering in response to a measured amount of solar energy, |
| Frost | = watering in response to frost contact input signal. |

## EXTERNAL CONTACT READINGS

READOUT  EXT

| External inputs 0-100% |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 41 25 20 38 36 45 41 49 28 56 45 25 57 46 52 60 |  |  |  |  |  |  |  |  |  |
|  |  | AUTO |  | PRINT |  |  |  |  | QUIT |

If the system is equipped with external contact signal inputs (16 shown above), this menu displays their values as percentages. The  PRINT  key provides an immediate printout of the current values.

### Periodic printout

READOUT  EXT  AUTO

| External inputs 0-100% |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Printout cycle : 0 min Archive interval : 0 min |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFY |  |  | QUIT |

| - Printout cycle ( 0  240 min. ) | Interval between two automatic printouts of the contact values. |
| --- | --- |
| - Archive interval ( 0  240 min ) | Interval between two automatic transfers of contact input data to the PC (logging). |

## STATION READINGS

STATION  READOUT

| Station readout 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible with pre-regulation option Visible when max. number of periods > 1 Setp Meas Mon Valve : Ec: 0.0 0.0 Time : pH: 1.0 1.0 Total : Ecp 0.0 Flow : l/h Ct: 1 2 3 4 0 % Volume : l St: --- Per : Total : l |  |  |  |  |  |  |  |  |  |
| SUN |  | FUTURE |  | OTHER |  | NEXT |  |  | QUIT |

This display contains input device readings, setpoint and accumulation values, together with status information.

| Setp (setpoint) | = values the system has been programmed to maintain (Ec, pH, ...): only visible during watering, |
| --- | --- |
| Meas (measurement) | = main sensor measurements (Ec, pH, ...). |
| Mon (monitor) | = monitor sensor measurements (Ec & pH). |
| Ct (contacts) | = status of the station's 4 'dry' contacts ("high" state indicated by a rectangle around the number): proportional contact measurement shown as 0..100 %, |
| St (station) | = current stage of watering, |
| Per (period) | = current period, |
| Valve (number) | = number(s) of the currently open valve(s), |
| Time (remaining) | = time remaining for the currently open valve or block of valves, |
| Total (time) | = total operating time of the station currently working, |
| Flow (l/h) - measurement | = current irrigation flow measurement. |
| Volume (l) - valve | = volume supplied by the valve or block of valves currently open, |
| Total (l) - volume | = total volume supplied during the irrigation. |

### More station readings

STATION  READOUT  NEXT

| Station readout 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature : 15°C Pump alarm : 10% Cnt 0-100% : 0% Tank empty alarm : 100% Detection 230V : 0% P / F / T alarm : 0% |  |  |  |  |  |  |  |  |  |
| HEATING |  | EXT INJ |  |  |  | VALVES |  |  | QUIT |

| Temperature | = station nutrient solution temperature |
| --- | --- |
| Cnt 0-100% | = status of station resistive contact |
| Detection 230V | = status of the 230V detection contact |
| Pump alarm | = status of the pump alarm contact |
| Tank empty alarm | = status of the tank empty alarm contact |
| P / F / T alarm | = status of the PFT alarm contact. |

## SOLAR ENERGY READINGS

READOUT  STATION  SUN

| Solarimeter |  |  | 12:05:00 |  |
| --- | --- | --- | --- | --- |
| 12h00 496 w/m2 Inst: 478 W/m2 Tot: 2990 Wh/m2 |  |  |  |  |
|  |  |  |  | QUIT |

The display graph provides access to readings covering a 24hr period that is continuously updated. Each mark on the horizontal axis represents 10 minutes. The vertical cursor is initially positioned at the current time: the graph of the current day's sunlight is displayed to the left and that for the previous day (at the same time of day) to the right. The position of the cursor can be changed using the direction arrows.

The cursor readings are shown to the right of the graph together with the following data:

| Inst (W/m2) | = current (instantaneous) sunlight intensity reading. |
| --- | --- |
| Tot (Wh/m2) | = accumulated solar energy recorded since midnight. |

## PROGRAMMED WATERING EVENTS READOUT

READOUT  STATION  FUTURE

| Programmed irrigation |  |  |  |  |  |  |  | 11:30:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time Group Start 12h30 1 Fixed time 13h30 2 Fixed time |  |  |  |  |  |  |  |  |  |
| DETAILS |  |  |  |  |  |  |  |  | QUIT |

The display lists the station watering events programmed for fixed times during the current day. The list is arranged in ascending order of start time and group. The 'DETAILS' key (F1) will display the relevant group Ec, inlet Ec and pH setpoints, as well as valve time or volume based control details.

### Readout of other programmed watering modes

READOUT  STATION  OTHER

| Programmed irrigation |  |  |  |  |  |  |  | 11:58:21 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group State Type Acti.time 1 0 Contact 12h10 2 248 Luxmeter 12h34 3 55 Contact 12h50 |  |  |  |  |  |  |  |  |  |
| DETAIL |  |  |  |  |  |  |  |  | QUIT |

The display lists the different station watering modes programmed for the current day at other than fixed times. The list contains only the next programmed activation of each group and start mode.

| State (current state) | = value depends on start mode, as follows: dry contact – value represents 0 or 1 (open or closed condition), proportional contact – value represents the measurement in percentage terms. solar energy start mode – value represents the corresponding accumulated energy measurement, cyclic mode – value is '0'. |
| --- | --- |
| Type (start mode) | = type of watering start mode programmed. |
| Acti. time (activation time) | = time at which each start mode is set to activate, taking into account the minimum intervals programmed for each mode. The estimated activation time for solar energy start mode is also based on the current sunlight measurement. |

## HISTORY

### Group running totals and averages

READOUT  GROUP

| Running totals and averages |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Gp No. Vol Time Ec EcA pH pHA 1 0 0 00:00:00 0.0 0.0 0.0 0.0 2 0 0 00:00:00 0.0 0.0 0.0 0.0 3 0 0 00:00:00 0.0 0.0 0.0 0.0 4 0 0 00:00:00 0.0 0.0 0.0 0.0 |  |  |  |  |  |  |  |  |  |
| HISTORY |  |  |  | PRINT |  |  |  |  | QUIT |

This display includes the daily running totals and averages for each group, in terms of the quantity and quality of the feed solution delivered.

| No. | = number of watering starts that have occurred, |
| --- | --- |
| Vol | = feed solution volume cumulative total, |
| Time | = total operating time, |
| Ec | = average Ec setpoint, |
| EcA | = average Ec reading, |
| pH | = average pH setpoint |
| pHA | = average pH reading. |

### Water starts

The  HISTORY  key in the previous display provides access to the last 100 water starts.

READOUT  GROUP  HISTORY

| Watering events |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DD MM Time ST Group Start Time Vol 1 04 15:00 1 2 Manual 00:03:00 150 2 04 14:00 1 3 Cyclic 00:05:00 250 |  |  |  |  |  |  |  |  |  |
| RESET |  | +4 |  | -4 |  |  |  |  | QUIT |

| RESET | = erase the contents of the list, |
| --- | --- |
| +4 / -4 | = scroll up or down by four lines. |

### Readout of water starts by valve

The display provides access to information covering the last 7 days.

READOUT  STATION  NEXT  VALVES

| Watering events |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Val Nr Time Volume Ec pH V-drn 1 3 00:09:00 3000 4.2 6.50 0 2 1 00:03:00 1000 4.2 6.5 0 3 0 00:00:00 0 0 0 0 4 2 00:10:00 1600 4.5 4.5 0 |  |  |  |  |  |  |  |  |  |
| -1 |  |  |  | PRINT |  |  |  |  | QUIT |

| Val | = number of the valve, |
| --- | --- |
| Nr | = number of water starts, |
| Time | = cumulative watering time, |
| Volume | = cumulative watering volume, |
| Ec | = average Ec reading, |
| pH | = average pH reading, |
| V-drn | = volume of drain water associated with the valve, |
| -1 | = view previous day readings. |

# ALARMS

## MAIN DISPLAY

ALARM

| Alarms menu |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Station Time Alarm 1 1 12h00 Ec |  |  |  |  |  |  |  |  |  |
| START |  | OFF |  | RESTART |  | MESSAGE |  |  | QUIT |

This menu displays any alarms the system may generate. The options include allowing the operator to restart the station, an action that also clears the alarm(s).

| START | = sets off the MCUFerti alarm buzzer, |
| --- | --- |
| OFF | = silences the MCUFerti buzzer, |
| RESTART | = restarts the station and group highlighted in the display by the horizontal bar: also clears down the alarm, |
| MESSAGE | = displays alarm messages, |
| Time | = time at which the alarm was detected, |
| Alarm | = cause of the alarm. |

### Causes of alarms

| Ec Alarm | = Ec measurement out of tolerance (Ec tolerance) for longer than the Ec alarm delay, |
| --- | --- |
| Max Ec alarm | = Ec measurement exceeds the Max Ec tolerance for longer than the Ec alarm delay, |
| Ec monitor difference alarm | = Ec monitor measurement exceeds the Ec monitor difference for longer than the Ec alarm delay, |
| pH alarm | = pH measurement out of tolerance (pH tolerance) for longer than the pH alarm delay, |
| Max pH alarm | = pH measurement exceeds the Max pH tolerance for longer than the pH alarm delay, |
| pH monitor deviation alarm | = pH measurement exceeds the pH monitor difference for longer than the pH alarm delay, |
| Pump alarm | = operation of the pump fault detection contact: this is constantly monitored during watering and the system will alarm if a fault persists for longer than the pump alarm delay, |
| Tank Empty alarm | = operation of the mixing tank low-level detection contact: except during the Emptying and Filling stages, the system will alarm if the tank empty delay (default 10 sec) is exceeded, |
| P/F/T alarm (Pressure/Flow/Temp.) | = operation of the Pressure/Flow/Temperature fault detection contact: the system will alarm if the P/D/T alarm delay (default 60 sec) is exceeded, |
| Power alarm (presence of power source) | = de-operation of the power source fault detection contact: after detection the occurrence of an alarm depends on the Power Supply alarm (default 'NO') and 220VAC alarm delay (default '0') settings. |
| Flow alarm | = flow rate measurement out of tolerance (Flow tolerance) or less than the Minimum flow setting for longer than the Flow alarm delay, |
| PIA alarm | = fault condition with one (or more) of the outputs on the MCUFerti I/O card: the main alarm is immediately activated affecting all stations controlled by the MCUFerti. |

## MESSAGE ALARMS

ALARM  MESSAGE

| Alarm messages |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Station Time Message 1 1 04h00 Min drn Ec |  |  |  |  |  |  |  |  |  |
|  |  | OFF |  |  |  |  |  |  | QUIT |

The menu displays details of any information message alarms.

| OFF | = silences the MCUFerti alarm buzzer. |
| --- | --- |
| Time (detection) | = time at which the alarm was detected, |
| Message (cause) | = cause of the message alarm. |

### Causes of message alarms

| Min drn Ec message | = drain water Ec measurement falls below the Min drainage Ec setpoint: the message is generated as soon as the 'irregularity' is detected, and is cleared once the measurement is greater than the setpoint. |
| --- | --- |
| Max drn Ec message | = drain water Ec measurement is greater than the Max drainage Ec setpoint: the message is generated as soon as the 'irregularity is detected, and is cleared when the measurement falls below the setpoint. |
| Min drn pH message | = drain water pH measurement falls below the Min drainage pH setpoint: the message is generated as soon as the 'irregularity' is detected, and is cleared once the measurement is greater than the setpoint. |
| Max drn pH message | = drain water pH measurement is greater than the Max drainage pH setpoint: the message is generated as soon as the 'irregularity' is detected, and is cleared when the measurement falls below the setpoint. |

# Manual commands

## OPERATION

MANUAL

| Manual programming |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Name Status Station 1 UNNAMED Free 1 2 UNNAMED Free 1 3 UNNAMED Free 1 4 UNNAMED Free 1 |  |  |  |  |  |  |  |  |  |
| START |  | STOP |  |  |  | ONE-OFF |  |  | QUIT |



Program single watering events

This display shows the current operating status of the groups.

| START | = starts a group irrigation immediately, |
| --- | --- |
| STOP | = stops the current group irrigation. |

The menu below illustrates the case of a group watering.

| Manual programming |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Name Status Station 1 UNNAMED Started 1 2 UNNAMED Free 1 3 UNNAMED Free 1 4 UNNAMED Free 1 |  |  |  |  |  |  |  |  |  |
| START |  | STOP |  | PAUSE |  | ONE-OFF |  |  | QUIT |

| PAUSE / RESTART | = pressing this button suspends the selected group: once pressed, the button function changes to RESTART allowing watering to continue when pressed. |
| --- | --- |

## PROGRAMMING A one-OFF WATERING START

MANUAL  ONE-OFF

| One-off starts |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Time Date 1 12h00 02/01/96 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFY |  |  | QUIT |

The display allows the operator to manually program one-off watering starts occurring at fixed dates and times. Unlike fixed time (time-of-day) starts described in the chapter on GROUPS, one-off starts disappear from the menu list once they have taken place.

Entering the number of a group automatically selects the current time and date as the one-off start time. These parameters can either be altered to generate a future event, or left unchanged which, when accepted, is the equivalent of a normal manual start.

- NOTE

The group must be correctly set up but does not need to be activated (see § 3.10.3).

# Drainage

This function involves the measurement of the amount of feed solution not absorbed by the plants during watering, which is expressed quantitatively as a percentage – the 'Drainage Rate' – calculated per group. The basic equipment includes a single flow meter per drainage line, plus optional Ec and pH sensors, as shown below.

Irrigation valve

MCU+

vol. watered

STATION

F

F

F

pH

Ec

Drain water tank

Line 1

Line 2

                    

               

drain water quality

vol. drained (1)

vol. drained (2)

## CONFIGURING THE MEASURING LINES

This requires level 4 (installation) permission or higher.

SYSTEM  INSTALL  OTHER  DRAIN

| Flow meter drainage 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of flow meter : FLMTR K factor : 1007 K factor exponent : -2 Volume per step : 1.0 l Volume per step : 0.0 ml Ec : 1 pH : 1 |  |  |  |  |  |  |  |  |  |
| N° DRN |  |  |  |  |  | MODIFY |  |  | QUIT |

Use this menu to specify the type and characteristics of the rate-of-flow meter in each line together with the numbers of the Ec and pH sensor inputs.

| - Type ( FLMTR / VLMTR ) | Specifies the type of rate-of-flow meter used for measurement: flow meters (FLMTR) produce a signal proportional to instantaneous flow, volumetric flow meters (VLMTR) give an output pulse for a given volume of fluid. |
| --- | --- |
| - K factor (i) ( 0  200000 ) & - K factor exponent (ii) ( 0  10 ) | Manufacturer's fitting factor for a flow meter (FLMTR). Together, these two parameters allow you to enter a number with a precision of 4 figures after the decimal point. Example: for a K factor = 10.07 (= 1007 x 10-2) 'K factor' = 1007, and 'K factor exponent' = -2. |
| - Vol per pulse (i) ( 0.0  1000.0 l) & - Vol per pulse (ii) ( 0.0  1000.0 ml) | Together these volumetric meter (VLMTR) parameters define the volume of liquid corresponding to one received pulse. Example: if 1 pulse corresponds to 6.21 liters, Vol per pulse (i) = 6.2, and Vol per pulse (ii) = 10 ml. |
| - Ec - pH ( 0  number of Ec and/or pH sensors in the installation) | Allocate specific Ec and pH sensor inputs to the drainage line. An entry of “0” means the measurement for the line in question will not be taken into account within the group. |

## SELECTING VALVES TO DRAINAGE LINES

Drainage measurements relate directly to irrigation valves and the groups to which they are assigned. Hence, programming valves for the drainage function is done through the menus concerned with groups and their valves, that is:

GROUP  VALVE  VALVES  FLOW

| Valve parameters |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Val. l/h/Dp N Dp m2 Fl Drn Drn Dp 1 0.0 0 0 0 0 0 2 0.0 0 0 0 0 0 3 0.0 0 0 0 0 0 4 0.0 0 0 0 0 0 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | ACCEPT |

Allocate each valve to a specific drainage line, and enter the number of drip emitters the valve supplies together with the number of emitters supplying the area under the collecting tray (see the simplified diagram and §3.6.5).

## DRAINAGE SETTINGS

GROUP  START  OTHER  DRAIN

| Drainage rate group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Drainage influences Visible with option max. number of periods > 1 Visible with drainage Ec option Visible with drainage pH option Time start drainage : 09:00 Time stop drainage : 18:00 Drainage rate : 0 % Max. solar energy influence : 1000W/m2 Min. solar energy influence : 500W/m2 Max % drainage deviation : 30 % Min. drainage Ec : 0.05 Ec Max. drainage Ec : 7 Ec Min. drainage pH : 3 pH Max. drainage pH : 7 pH |  |  |  |  |  |  |  |  |  |
| PERIOD |  | INFLU. |  |  |  | ACCEPT |  |  | QUIT |

	

| - Drainage rate (0  100%) | Required objective, where: Drainage rate ( %) = (vol. drained / vol. irrigated ) x 100. |
| --- | --- |
| - Max. solar energy influence ( Min. energy influence  1000 W/m2 ) | Threshold value of solar energy corresponding to the maximum increase in 'Drainage rate' defined by the "Max. drainage rate modification" setting. |
| - Min. solar energy influence ( 0 W/m2  Max. energy influence ) | Threshold value of solar energy corresponding to the minimum (zero) increase in the 'Drainage rate'. |
| - Max. drainage rate modification ( 0  100 %) | Maximum increase in the 'Drainage rate' due to solar energy. The increase is proportional to the solar energy measurement over the programmed range of influence. Can be programmed for the periods accessible using the PERIOD key (F1). |
| - Min / max drainage Ec - pH ( 0 / 1  10 / 9 ) | Minimum and maximum Ec and pH limits. Over or under-limit readings generate an alarm message (this does not cause the station to stop but informs the grower of a limit violation). |

## SUMMARY OF THE DRAINAGE RATE CALCULATION

Drainage rate is calculated per group, and is the relationship between the volume drained and the volume watered by the valves for any given group, where:

(a) volume 'drained' for a valve =

n° of emitters supplied

vol. drained by the line  x   --------------------------------------------------------

n° of drainage line (collecting tray) emitters

(b) total volume drained for a group  = sum of volumes drained by all valves in the group.

volume drained by the group

(c) drainage rate	 = 	------------------------------------

volume watered by the group

Therefore, to ensure accuracy the number of emitters each valve supplies together with the number of emitters watering over the collecting tray must be programmed correctly.

- NOTE:

The volume drained by a line cannot be equal to the volume drained by the corresponding group unless the group has only one valve for which the number of watering and drainage drip emitters is the same.

## DRAINAGE READINGS

### Group readings

READOUT  DRAIN

| Drainage per day |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Ir.Vol Dn.Vol % Ec pH 1 200 150 15 9.0 4.0 2 0 0 0 0.0 1.0 3 0 0 0 0.0 1.0 4 0 0 0 0.0 1.0 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | PRINT |  |  |  |  | QUIT |



Printout of drainage values

| Ir.Vol (litres) | = volume of water supplied during an irrigation: equal to the sum of the volumes of all group valves assigned to a given drainage line. |
| --- | --- |
| Dn.Vol (litres) | = volume of water drained: equals the sum of the measurements (volumes) multiplied by a weighting factor given by: no. of valve emitters / no. of emitters for drainage. (if the number of drainage emitters is zero, this volume equals the sum total of the drainage measurements), |
| % | = Drainage rate, which equals: (volume drained / volume watered) x 100. |
| Ec - pH | = average Ec and pH values of the drain water (measured over a period of 1 minute). |
| PRINT | = printout of the running totals (from midnight) of water drained, irrigation volume and drainage rate for all MCU Ferti groups. |

### Drainage line readings

READOUT  DRAIN DRAIN

| Drainage |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| N° V.Drn COD Ec CEc pH CpH 1 0 0 0 0.0 1.0 1.0 2 0 0 0 0.0 1.0 1.0 3 0 0 0 0.0 1.0 1.0 4 0 0 0 0.0 1.0 1.0 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | QUIT |

| N° | = number of the drainage line |
| --- | --- |
| V.Drn | = accumulated volume of water drained by the line |
| COD | = COD |
| Ec | = average drainage Ec value calculated over a period of 1 minute, |
| CEc | = accumulated average drainage Ec value calculated over a period of 1 minute (reading used by the installer for checking purposes), |
| pH | = average drainage pH value calculated over a period of 1 minute, |
| CpH | = accumulated average drainage Ec value calculated over a period of 1 minute (reading used by the installer for checking purposes). |

## DRAINAGE INFLUENCES ON WATERING

To optimize watering you can program drainage rate influences on the following group parameters:

- radiation start trigger measurement (Wh/m²),

- proportional contact start trigger measurement (%),

- cyclic irrigation interval (min).

Access for programming is as follows:

GROUP   START  OTHER  DRAIN  INFLU.

| Drainage influences group 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rad. correction : 20 Wh/m2/% Max rad. threshold : 50 Wh/m2 Min rad. threshold : 0 Wh/m2 Contact correction : 10 %/% Max cnt threshold : 50 %/% Max cnt threshold : 0 %/% Cyclic correction : 25 Min/% Max cycle interval : 15 Min/% Max cycle interval : 5 Min/% |  |  |  |  |  |  |  |  |  |
| PERIOD |  |  |  |  |  | ACCEPT |  |  | QUIT |

Influences

| - Rad. correction ( 0  50 Wh/m²/% ) | Change to the solar radiation start threshold for each percentage difference between the drainage rate objective and the measurement. |
| --- | --- |
| - Contact correction. ( 0  50 % / % ) | Change to the proportional contact start threshold for each percentage difference between the drainage rate objective and the measurement. |
| - Cyclic correction ( 0  50 min / %) | Change to the cycle interval for each percentage difference between the drainage rate objective and the measurement. |

The corrections are calculated each time a group irrigation takes place.

Limits

Influence calculations based on false measurements (due to an installation fault, for example) can soon lead to abnormal watering. For this reason, the remaining menu settings enable you to limit the changes to the influenced parameters. The settings are explained overleaf.

| - Max & min radiation ( 0  9999 Wh / m² ) | The trigger threshold for radiation starts is influenced only by energy measurements within the minimum and maximum energy range. |
| --- | --- |
| - Max & min contact ( 0  100 % ) | The trigger threshold for contact starting is influenced only by proportional contact measurements within the minimum and maximum percentage range. |
| - Max & min cyclic ( 0  1440 min ) | The corrected cyclic watering interval is within the maximum and minimum time range. |

### Conclusion

In order to enable drainage influences on a group program the following:

- activation of the group

- activation of the start modes (fixed time, cyclic, solar energy, contact)

- activation periods (start and stop times)

- drainage influences

- maximum and minimum limits for drainage influences.

- NOTES:

(i) Group start parameters influenced by drainage are calculated automatically. Any changes to these parameters made manually cancel the drainage influence until the next watering.

(ii) The fact of dividing the day into periods does not alter drainage rate measurements made across periods: only the drainage rate objective and start influence settings are affected.

# STATION WATER PRE-HEATING

Irrigation

Heating                   water T°

water T°

Heat exchanger

Water source

Pre-heating maintains water supplied to the mixing tank at a constant temperature. The system includes a heat exchanger in a typical 3-way valve and pump arrangement together with sensors for the heating circuit and tank inlet water temperature measurements.

The apparatus is connected to a classic MCU watering station, which manages temperature by regulating the mixing valve and pump as part of its overall control of the station.

## HEATING SETTINGS

SYSTEM  INSTAL  OTHERS	 HEATING

Enable water heating : YES

Min. watering temp. for alarm: 10 C

Irrigation water temp.: 18 C

Max. watering temp. for alarm: 25 C

Min. heating water temp.: 45 C

Max. heating water temp. : 60 C

Alarm delay time: 60 sec

CONFIG

MODIFY

QUIT

Heating settings 1

10:12:15

| - Enable water heating (YES / NO) | Allows or inhibits the water pre-heating function. |
| --- | --- |
| - Min. watering temp. for alarm ( 0 / 130 °C ) | Irrigation water low temperature threshold below which the controller generates an alarm. |
| - Irrigation water temp. ( 0 / 130 °C ) | Irrigation water setpoint temperature. |
| - Max. watering temp. for alarm ( 0 / 130 °C ) | Irrigation water high temperature threshold which, if exceeded, causes a controller alarm. |
| - Min. heating water temp. ( 0 / 130 °C ) | Minimum water temperature to be maintained in the heating circuit (see ‘Min. temperature difference control 3WV’ further on). |
| - Max. heating water temp. ( 0 / 130 °C ) | Maximum permitted heating circuit water temperature (see ‘Minimum temperature difference 3WV' control). |
| - Alarm delay time ( 0 / 3600 sec ) | Period for which the detected alarmable condition must remain uninterrupted before the alarm annunciates. |

## CONFIGURATION

SYSTEME  INSTAL  AUTRE  CHAUFF  CONFIG

Enable water heating : OUI

Heating on station number: 1

3WV control interval: 2 *5ms

Stop heating if no irrigation: YES

Water temperature difference 3WV control: 0.5C

Minimum 3WV control time per degree C: 1.0sec

Minimum temperature difference 3WV control: 0.5C

Maximum 3WV control time per degree C: 5.0sec

Maximum temperature difference 3WV control: 5.0C

3WV overall control time: 60sec

Maximum change in position of V3V: 60%

Time maintain V3V position:  0sec

Difference position 3WV for safety: 10%

Pump stop delay: 60sec

Initial position 3WV: 33%

CONFIG

MODIFY

QUIT

Heating settings 1

10:12:15

| - Heating on station number ( 1 / 8 ) | Number of the station to which the pre-heating system is connected. |
| --- | --- |
| - 3WV control interval ( 1 / 240 *5ms ) | Pre-heating temperature control interval. |
| - Stop heating if no irrigation ( OUI / NON ) | If this parameter is set to ‘YES’ the pre-heating system is only controlled when the station to which it is linked is watering: when the station stops it closes the 3-way valve and turns the pump OFF. |
| - Water temperature difference 3WV control ( 0 / 130 °C ) | Temperature difference between the irrigation water setpoint and the measurement allowing control corrections to the 3-way valve. |
| - Minimum 3WV control time per degree C ( 0 / 3600 sec ) | Minimum 3-way valve control time for a 1 (one) degree difference between the irrigation water temperature setpoint and the measurement. |
| - Minimum temperature difference 3WV control ( 0 / 130 °C ) | Temperature difference between the irrigation water setpoint and the measurement corresponding to the minimum control time defined by the previous setting. |
| - Maximum 3WV control time per degree C ( 0 / 3600 sec ) | Maximum 3-way valve control time for a 1 (one) degree difference between the irrigation water temperature setpoint and the measurement. |
| - Maximum temperature difference 3WV control ( 0 / 130 °C ) | Temperature difference between the irrigation water setpoint and measurement corresponding to the maximum control time defined by the previous setting. |

In order to correct for varying setpoint-to-measurement temperature differences, the last four settings configure control of the 3-way valve in terms of ‘3WV control time per °C’.

Example:

Irrigation water temperature (setpoint)			: 18 °C

Minimum 3WV control time per degree C		: 1.0 sec

Minimum temperature difference 3WV control		: 0.5 °C

Maximum 3WV control time per degree C		: 5.0 sec

Maximum temperature difference 3WV control		: 3.0 °C

Temperature table (measurement /difference)

| Water T° meas. (°C) | 14 | 15 | 16 | 16.5 | 17 | 17.5 | 18 | 18.5 | 19 | 19.5 | 20 | 21 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3WV control T° diff. (°C) | 4 | 3 | 2 | 1.5 | 1 | 0.5 | 0 | 0.5 | 1 | 1.5 | 2 | 3 | 4 |
| Control time / °C (sec) | 5 | 5 | 3.4 | 2.6 | 1.8 | 1 | 1 | 1 | 1.8 | 2.6 | 3.4 | 5 | 5 |

where in the table above:

- ‘Water T° meas.’ is the irrigation water temperature measurement,

- ‘3WV control T° diff.’ is the difference between the water temperature measurement and the setpoint,

- ‘Control time per °C’ is the final calculated control time per °C for the 3-way valve.

0

(min)                                          (max)                   Control T° diff.

Calculated control time /°C

(max)

(min)

The graph shows that control based on the calculation of ‘3WV control time /°C’ varies linearly between minimum and maximum values over the permitted temperature difference range.

Configuration settings continued

| - 3WV overall control time ( 0 / 3600 sec ) | Time taken for the 3-way valve to move from fully closed (0%) to fully open (100%). |
| --- | --- |
| - Maximum change in position of 3WV ( 0 / 100 % ) | Maximum change in the position of the 3-way valve per control interval: this setting restricts the amount of variation in the valve’s position. |
| - Time maintain V3V position ( 0 / 3600 sec ) | Period of time the 3-way valve is held at its initial position when watering starts. |
| - Difference in position of 3WV for safety ( 0 / 100 % ) | Percentage added to or subtracted from the position of the 3-way valve when the heating water temperature violates either the maximum or minimum temperature threshold. |
| - Pump stop delay ( 0 / 3600 sec ) | Defines the time that elapses before the command to stop the pump takes effect. |

## READINGS

This display enables you to monitor the status of the pre-heating system, including the regulation of the 3-way valve and pump.

READOUT  STATION  NEXT  HEATING

| Heating readout |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Irrigation T° Heating water T° Ctrl 18.0 Pos 3WV 55% Meas 18.2 55.0 Pump status YES Min 45.0 Station YES Max 60.0 Heating ON Ctrl time 1.0sec |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | QUIT |

| Ctrl | = water temperature setpoint, |
| --- | --- |
| Meas | = water temperature measurement, |
| Min | = minimum water temperature. |
| Max | = maximum water temperature. |
| Pos. V3V | = heating 3WV position, |
| Pump status | = pump ON or OFF, |
| Station | = number and status of the station to which the pre-heating system is connected, |
| Heating | = heating status ON or OFF, |
| Ctrl time | = 3WV control time per °C calculated by the MCU Plus. |

# PRINT MENUS

Printouts of the settings programmed for stations, groups, valves, drainage and other functions can be obtained on a dot matrix printer connected to the MCU. Station event lists can be printed automatically for each irrigation, and a print out of daily statistics day can be programmed to occur at a fixed time during the day.

## PROGRAMMED SETTINGS

The following menu enables some or all of the main settings programmed for the functions of the MCU+ to be printed.

SYSTEM  OPTIONS  PRINT

| Print programmed settings |  |  |  |  |  |  |  | 10 :12 :15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Print : YES Group settings : 1 to 8 Station settings : 1 to 1 Valve settings : 1 to 20 Drainage settings : 1 to 2 Heating settings : 1 to 1 |  |  |  |  |  |  |  |  |  |
| PRINT |  |  |  | CONFIG |  | MODIFY |  |  | QUIT |

On each line the values entered define the printout range for the named settings. Entering '0' cancels the printout.

| - Print (Yes /No) | Enable settings to be printed out. |
| --- | --- |
| - Group settings (- to -) | Print the main settings for a range of groups. |
| - Station settings (- to -) | Print the main settings for a range of stations. |
| - Valve settings (- to -) | Print the main settings for a range of valves. |
| - Drainage settings (- to -) | Print the main drainage settings. |
| - Heating settings (- to -) | Print the heating settings. |
| PRINT | = press to start a printout the selected functions (key appears if the 'Print' setting has been set to 'YES'). |

## CONFIGURING STATION PRINTOUTS

SYSTEM  OPTIONS  PRINT  CONFIG  STATION

| Station printout |  |  |  |  |  |  |  | 10 :12 :15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Flow : YES Inputs : NO Installation parameters : YES Nutrient counter : NO Irrigation cart : NO |  |  |  |  |  |  |  |  |  |
| PRINT |  |  |  |  |  | MODIFY |  |  | QUIT |

Use this menu to select the settings to be printed. The categories shown correspond to those found in the station configuration menus (SYSTEM  INSTAL  STATION).

Printouts consist only of the chosen categories once the menu entries have been saved.

| PRINT | = opens a menu allowing you to print the settings for a specific station. |
| --- | --- |

## CONFIGURING GROUP PRINTOUTS

SYSTEM  OPTIONS  PRINT  CONFIG  GROUP

| Groups printout |  |  |  |  |  |  |  | 10 :12 :15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EC/PH settings : YES Valve settings : YES Start settings : YES General settings : YES Install.settings : NO |  |  |  |  |  |  |  |  |  |
| PRINT |  |  |  |  |  | MODIFY |  |  | QUIT |

Use this menu to select the settings to be printed. The categories correspond to those found in the group programming and installation menus.

Printouts consist only of the chosen categories once the menu entries have been saved.

| PRINT | = opens a menu for printing the settings for a specific group. |
| --- | --- |

## AUTOMATIC PRINTOUTS

Automatic station printouts at each start event include the start time, pause or alarm times (if any), stop time, and watering statistics. Automatic daily printout of statistics for groups, valves, drainage and nutrients can be programmed to occur at a given fixed time of the day.

SYSTEM  OPTIONS  PRINT  CONFIG  AUTO

| Printout of groups |  |  |  |  |  |  |  | 10 :12 :15 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Automatic printouts : YES Time of printout : 23:59 Printout of station : YES Printout of groups : YES Printout of valves : YES Printout of drainage : YES Printout of nutrients : YES |  |  |  |  |  |  |  |  |  |
| STATION |  | GROUP |  | VALVE |  | MODIFY |  |  | QUIT |

| - Automatic printouts (Yes/No) | Enable this function. |
| --- | --- |
| - Time of printout (00:00 - 23:59) | Time at which the daily statistical printout occurs. Caution: (i) accumulated daily values are reset to '0' at midnight and, (ii) any water start occurring after the printout is not included in the printout for the next day. |
| - Printout of station (Yes/No) | Enable automatic station printouts. |
| - Printout of groups (Yes/No) | Enable automatic group printouts. |
| - Printout of valves (Yes/No) | Enable automatic valve printouts. |
| - Printout of drainage (Yes/No) | Enable automatic drainage printouts. |
| - Printout of nutrients (Yes/No) | Enable automatic printout of nutrient counter values. |

The  STATION ,  GROUP  and  VALVE  keys can be used to cancel automatic printouts for a specific station, group or particular valve, however:

- although changes in the status of  the station (start, stop, and pause) are cancelled, any station alarms will still be printed, and

- canceling printouts for group that starts a station also cancels the printout for the station.

# INSTALLATION PARAMETERS

## SYSTEM / INSTALL MENUS

SYSTEM

| System parameters |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Friday 01 January 1998 : 08h34 : 17h29 |  |  |  |  |  |  |  |  |  |
| INSTALL |  | OPTIONS |  | DAT/TIM |  | CODE |  |  | QUIT |



Installation parameters

Installation parameters are accessed through the main menu  SYSTEM  key.

The  INSTALL  option, only visible when the system is accessed at level 3 or higher, provides access to the configuration possibilities shown below.

SYSTEM  INSTALL

| Installation parameters |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TEST |  | GROUP |  |  |  | STATION |  |  | QUIT |

			

Group configuration

Station configuration

Test menus & displays

## INPUT / OUTPUT TEST DISPLAYS

SYSTEM  INSTALL  TEST

| Installation tests |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INPUTS |  | OUTPUTS |  | COUNTER |  |  |  |  | QUIT |

		

Readings

Test & readings

Same as for inputs but numbers between 0 & 255 (no 'board' key)

These menus allow you to check the correct operation of the system's inputs and outputs.

### Inputs display

SYSTEM  INSTALL  TEST  INPUTS

| Inputs |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 450 523 121 470 854 465 580 662 257 234 481 456 198 892 524 449 451 875 855 493 534 517 571 660 535 805 413 498 566 909 143 292 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | BOARD |  |  |  |  | QUIT |

The 32 values in the display correspond to measurements made on one of the input/output cards. They are expressed as numeric (digital) values lying between 0 and 1023.

To display the next group of inputs, another I/O board must be selected using the  BOARD  key.

| Inputs |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 450 523 121 470 854 465 580 662 257 234 481 456 198 892 524 449 I/O board number: 1 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | BOARD |  | ACCEPT |  |  | QUIT |

### Testing outputs

SYSTEM  INSTALL  TEST  OUTPUT

| Installation tests |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Manually activate output : 1 |  |  |  |  |  |  |  |  |  |
| STOP |  | START |  |  |  | STATE |  |  | ACCEPT |

Manual testing of the outputs is controlled by the  START  and  STOP  options. Once activated outputs automatically turn off after 5 seconds.

### Output states

SYSTEM  INSTALL  TEST  OUTPUT  STATE

| Outputs |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | BOARD |  |  |  |  | ACCEPT |

The 32 values in the display correspond to the state of the outputs. A value of '0' indicates that the output is off, a value between 1 and 128 indicates an active output.

## CONFIGURING GROUPS

SYSTEM  INSTALL  GROUP

| Choice of group |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Group Name Active Station 1 UNNAMED NO 1 2 UNNAMED NO 1 3 UNNAMED NO 1 4 UNNAMED NO 1 |  |  |  |  |  |  |  |  |  |
|  |  | TEST |  | PARAM |  |  |  |  | QUIT |

	

Station status readings

Group parameters

Move the horizontal bar in the display to select a group. The  TEST  key (F2) provides access to the station parameters display, which allows direct stop/start control of the station. The  PARAM  option (F3) provides access to a menu for entering group installation parameters.

## GROUP REGULATION PARAMETERS

SYSTEM  INSTALL  GROUP  PARAM

| Group parameters 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station : 1 Kp Ec : 1.0 KpR Ec : 0.5 Kp Ec tolerance : 0.5 Ki Ec : 1.0 Kd Ec : 0.5 Min injection time Ec : 30 x10ms Max injection time Ec : 500 x10ms Initial injection time Ec : 0 x10ms injection time Ec prereg : 30 x10ms Kp Ecp : 0.8 Kd Ecp : 0.5 Kp pH : 0.3 KpR pH : 0.2 Kp pH tolerance : 0.5 Ki pH : 1.0 Kd pH : 0.5 Min injection time pH : 30 x10ms Max injection time pH : 500 x10ms Initial injection time pH : 0 x10ms Ec/ph pre-mix offset : 0 sec Ec pH maintain time : 0 sec Modify Ec pH station ON : NO |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFY |  |  | QUIT |

The nutrient parameter values in this display, per group, are permanently stored in memory. Any changes will take effect when the new values are transmitted to the station at the next watering start. Use this menu to make permanent any parameter values tested using the menu in §10.5 since test values are lost once watering has ended.

| - Station ( 1  No. of system stations ) | Number of the station to which the irrigation group belongs. |
| --- | --- |
| - Kp Ec ( 0.0  10.0 ) | Represents a proportional correction (amplification) to Ec regulation. Together with the control difference obtained from the equation Ec difference = Ec setpoint - Ec measurement, it is used to calculate the product KpEc x Ec difference in order to give a injection time proportional to the control difference. Increasing KpEc leads to faster regulation but increases the risk of instability. Reducing its value increases stability at the expense of a slower reaction time. The parameter takes effect for Ec measurements outside the range Ec setpoint ± Ec tolerance. |
| - KpR Ec ( 0.0  10.0 ) | Identical to the previous parameter, except that it acts when Ec measurements are within the range given by Ec setpoint ± Ec tolerance. |
| - Kp Ec tolerance (0-10) | Defines the tolerance range for the Kp and KpR parameters described above. |
| - Ki Ec ( 0.0  1.0 ) | Represents an integral correction factor applied to Ec regulation. Its role is to absorb the permanent error resulting from the proportional correction. KiEc is independent of control differences, and is used to calculate the product KiEc x Old nutrient injection time, which is then added to the next injection time, and also ensures the system continues to demand nutrient even when the control difference is zero. In practice, a value of Ki Ec = 1.0 gives satisfactory control. |
| - Kd Ec ( 0.0  10.0 ) | Represents a derived correction to Ec regulation. Its role is to predict undesirable variations. The system calculates the derivative (rate of change) of the control difference using the product KdEc x ( Ec difference - old Ec difference ) the value of which is added to the injection command, acting to oppose any large increases. This approach effectively 'damps' control difference (and hence measurement) variations, leaving Ec regulation under mainly proportional and integral control. |
| - Min injection time Ec ( 0  500 x10ms ) | Fixes the injection time lower limit. Calculated injection times (for EC control) smaller than the value entered are ignored, but are taken into account in the next calculation. |
| - Max injection time Ec ( 0  500 x10ms ) | Fixes the injection time upper limit. The injection time is forced to this value if the calculated injection time (for EC control) is greater. |
| - Initial injection time Ec ( 0  500 x10ms ) | The initial nutrient /acid (or base) injection time applied when the group starts, and used as the starting point for control. |
| - Injection time Ec prereg ( 0  500 x10ms ) | Fixes the injection time lower limit. Calculated injection times (for EC pre-regulation control) smaller than the value entered are ignored, but are taken into account for the next calculation. |
| - Kp Ecp ( 0.0  10.0 ) | Represents a proportional correction (or amplification) to the process of Ec pre-regulation by 3-way valve. The principle is identical to that applying to normal Ec control. |
| - Kd Ecp ( 0.0  10.0 ) | Represents a derived correction to the process of Ec pre-regulation by 3-way valve. The correction principle is identical to that applying to normal Ec control. |
| - Kp pH ( 0.0  10.0 ) | Represents a proportional correction (amplification) to pH regulation. The correction principle is identical to that applying to the control of Ec. |
| - KpR pH ( 0.0  10.0 ) | Identical to the previous parameter, except that it acts when pH measurements are within the range given by pH setpoint ± pH tolerance. |
| - Kp pH tolerance (0-10) | Defines the tolerance range for the Kp and KpR parameters described above. |
| - Ki pH ( 0.0  1.0 ) | Represents an integral correction factor applied to pH regulation. The correction principle is identical to that applying to the control of Ec. |
| - Kd pH ( 0.0  10.0 ) | Represents a derived correction to the pH regulation. The correction principle is identical to that applying to the control of Ec. |
| - Min injection time pH ( 0  500 x10ms ) | Fixes the injection time lower limit. Calculated injection times (for pH control) smaller than the value entered are ignored, but are taken into account for the next calculation. |
| - Max injection time pH ( 0  500 x10ms ) | Fixes the injection time upper limit. The injection time is forced to the value entered if the calculated injection time (for pH control) is greater. |
| - Initial injection time pH ( 0  500 x10ms ) | Initial nutrient /acid (or base) injection time applied when the group starts, and used as the starting point for control. |
| - Ec/pH pre-mix offset ( 0  30 sec ) | Represents the time delay between the start of Ec correction and the start of pH correction during the pre-mixing stage. |
| - Ec pH maintain time ( 0  30 sec ) | Determines the period for which fixed injection times are maintained following the start of regulation. |
| - Modify Ec pH station ON ( Yes / No) | Set this parameter to 'Yes' to cause any modifications to the group's Ec and pH settings take effect immediately, even if watering is in progress. |

## IRRIGATION TEST MENUS

SYSTEM  INSTALL  GROUP  TEST

| Station readout 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station regulation parameters Visible with nutrient counter option Visible with periods option Setp Meas Inj Valve : Ec: 0.0 0 Time : 0 Total : pH: 1.0 0 Flow : l/h 0 Volume: l PER : Total : l |  |  |  |  |  |  |  |  |  |
| STOP |  | START |  | Kp / Ki |  | NUTRIE. |  |  | QUIT |

	

This display gives the same information as the normally accessed station readout, however, there are several important differences:

- the true values of Ec and pH (shown in mS/cm and pH) are displayed with their digital values (internal to the MCUFerti) shown underneath,

- the Ec and pH monitor measurements are replaced by nutrient and acid/base  injection times (x10ms),

- the START (F2) and STOP (F1) options allow the station group to be directly controlled,

- this means that the process of nutrient regulation can be monitored and adjusted during the fertilization stage. The Kp/Ki option provides access to the station's nutrient regulation parameters.

SYSTEM  INSTALL  GROUP  TEST  Kp/Ki

| Station parameters 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Kp Ec : 1.0 KpR Ec : 0.5 Ki Ec : 1.0 Kd Ec : 0.5 Kp Ecp : 0.8 Kd Ecp : 0.5 Kp pH : 0.3 KpR pH : 0.2 Ki pH : 1.0 Kd pH : 0.5 Ec/pH pre-mix offset : 0 sec Ec pH maintain time : 0 sec |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ACCEPT |  |  | QUIT |

The station parameters in this display are 'live', any changes made during watering taking effect immediately. In this way, parameters can be tested to determine suitable values while the station is active. However, these entries are lost once watering has finished. To make tested parameter values permanent you must then enter them through the menu described in §10.4.

## STATION CONFIGURATION

SYSTEM  INSTALL  STATION

| Station configuration |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Station Type No. nutrients 1 IWAKI 4 |  |  |  |  |  |  |  |  |  |
| FLOW |  | INPUTS |  | GENERAL |  | ACCEPT |  |  | QUIT |

		

Station inputs

Station flowmeter

Station parameters

This display shows the station(s) over which the MCUFerti computer has control. Move the horizontal bar to highlight the station to be configured.

### Flow meters

SYSTEM  INSTALL  STATION  FLOW

| Flow meter station 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type of flow meter : FLMTR K factor : 1007 K factor exponent : -2 Volume per step : 1.0 l Volume per step : 0.0 ml Ec : 1 pH : 1 |  |  |  |  |  |  |  |  |  |
| N° DRN |  |  |  |  |  | MODIFY |  |  | QUIT |

| - Type ( FLMTR / VLMTR ) | Specifies the type of rate-of-flow meter used for measurements: flow meters (FLMTR) produce a signal proportional to instantaneous flow, volumetric flow meters (VLMTR) give an output pulse per given volume of fluid. |
| --- | --- |
| - K factor (i) ( 0  200000 ) & - K factor exponent (ii) ( 0  10 ) | Manufacturer's fitting factor for a flow meter (FLMTR). These two parameters combined allow you to enter a number with a precision of 4 figures after the decimal point. Example: for a K factor = 10.07 (= 1007 x 10-2) 'K factor' = 1007, and 'K factor exponent' = -2. |

| - Vol per pulse (i) ( 0.0  1000.0 l) & - Vol per pulse (ii) ( 0.0  1000.0 ml) | Together these volumetric meter (VLMTR) parameters define the volume of liquid corresponding to one received pulse. Example: if 1 pulse corresponds to 6.21 liters, Vol per pulse (i) = 6.2, and Vol per pulse (ii) = 10 ml. |
| --- | --- |

### Station inputs

SYSTEM  INSTALL  STATION  INPUTS

| Station inputs 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible with 3-way valve Ec pre-regulation option Main Ec measurement : YES Ec monitor measurement : YES Main pH measurement : YES pH monitor measurement : YES Power supply : YES Luxmeter : YES Flowmeter : YES Temperature : YES Contact 0-100% : YES Pump : YES Tank empty : YES P/F/T (pressure, flow, T°) : YES Dry contacts : YES Dry contacts : YES Dry contacts : YES Dry contacts : YES Inlet Ec : YES |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ACCEPT |  |  | QUIT |

| - Input xxx ( YES  NO ) | Specifies whether station input 'XXX' is active or not. Each active input 'YES' occupies a physical input in the order defined during manufacturing. Note: the P/F/T alarm input that can be assigned to any contact. When the contact is HIGH (closed) the station will alarm and stop. |
| --- | --- |

## GENERAL STATION PARAMETERS

SYSTEM  INSTALL  STATION  GENERAL

| Station parameters 1 |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Visible with 3-way valve Ec pre-regulation option pH regulation type : ACID Min. stop time : 0 sec No. valves per group : 16 Automatic restart (YES/NO) : NO Power supply alarm : NO Pump alarm delay : 0 sec Power supply alarm delay : 0 sec P/F/T alarm delay : 60 sec Tank empty alarm delay : 10 sec Solarimeter gain : 100 % 3W valve inlet change over : 0 sec |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | ACCEPT |  |  | QUIT |

| - pH regulation type ( ACID / BASE ) | Specifies the type of pH correction to be used, on which the calculation of injection time is then based. |
| --- | --- |
| - Min. stop time ( 0  300 sec ) | Minimum interval between two successive water starts. The station pumps are not normally turned off at the end of watering if the system detects that another group is waiting to start. However, if the 'minimum stop time' setting is greater than zero the station will automatically stop. |
| - No. valves per group ( 16  Number of valves in the system: 64 max ) | Maximum number of valves programmable per group. This is a general parameter applying to all station groups. |
| - Automatic restart ( YES/NO ) | 'NO' specifies that the station is put into alarm status if one of the following alarmable conditions is detected: tank empty, P/F/T, pump fault, or power loss. Otherwise, for the same conditions, a 'YES' entry causes the station to pause and then restart automatically when the alarm condition disappears. |
| - Power supply alarm ( YES/NO ) | Enables or inhibits the power supply alarm (presence of 230VAC detected within the MCU+ Ferti). |
| - XXXX alarm delay ( 0  120 sec) | Programmable time delay before activation of the XXXX alarm (any of the 4 alarms in the menu). Caution : a setting of '0' disables the alarm. |
| - Solarimeter gain ( 0-999 ) | Solar energy measurement correction factor (offset). |
| - 3W valve inlet change over ( 0  400 sec ) | For systems equipped for Ec pre-regulation, this parameter represents the time taken for the 3-way valve to fully open or close. |

## DEFINING GROUP AND STORED CONTACTS

SYSTEM  INSTAL  OTHER  CONTACT

| Configuration contact |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| St Cnt Type Thresh Inver Status 1 1 STORED 50% OUI 1 1 2 0-100% 50% NON 0 0 1 0-100% 50% NON 0 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFY |  |  | QUIT |

| - St (1 – no. of stations) | Number of the station. |
| --- | --- |
| - Cnt (1 – no. of contacts) | Number of the station contact. Setting a contact number to '0' disables its function if it has a stop, suspend or next valve action. |
| - Type (0-100% / stored) | Type of contact, ‘0 –100%’ or ‘stored’. A 0–100% contact operates and de-operates for measurements above and below the threshold, respectively. A stored contact operates when the measurement exceeds the threshold, its status remaining unchanged even if the signal level drops below the threshold (for example, a pulse input). The reset-to-zero conditions for contacts of the stored type depend on the function programmed for the contact, as in the following table. |

| Stored contact function  | Start | Stop | Next valve |
| --- | --- | --- | --- |
| Resets to '0' when  | (i) Group stops (ii) Activation or deactivation of the group | (i) Group stops (ii) Activation or deactivation of the group | (i) Group starts (ii) Control passes to next valve (iii) Activation or deactivation of the group |

| - Thresh ( 0 – 100% ) | Threshold measurement above which the contact operates. This parameter is only valid if the contact is used for the stop, suspend and next valve functions, since the threshold for a start contact is defined in the GROUP  START  CONTACT menu (section §3.7.3). |
| --- | --- |
| - Inver (Yes /No) | Inverts the input measurement and the operating sense of the contact. Normally ('NO'), the numerical value of the input used by the program is proportional to the measurement signal. If inversion is selected ('Yes'), the numerical value is inversely proportional to the measurement, and the operating sense of the input is inverted (contact operates below the threshold). |

## MULTIFUNCTION CONTACTS

This term refers to MCU+ inputs that can be configured for a range of sensors (see the description of the 'Sen' column below), and which can have the following uses: provide sensor readings; stop or suspend a group; frost contact start a group.

In the menu below the entries in the minimum, maximum and sensor type columns calibrate the measurement readings for the corresponding sensors numbered on the left.

SYSTEM  INSTAL  OTHER  CFG CNT

| Config contact ext |  |  |  |  |  |  |  | 00:00:00 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NO Meas Min Max Cal Sen Read 1 512 0 130 0 NTCE 40.0 2 403 0 1000 0 5-25 200.0 3 10 0 100 0 CNT 0.1 4 1023 0 100 0 0-4 100 |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | MODIFY |  |  | QUIT |

The list of possible entries in the ‘Sen’ column includes:

5-25	(mA current loop); 0-4 (mA current loop); 0-20 (mA current loop); 4-20 (mA current loop);CNT (dry contact); NTCA (ELOTEC NTC ambient temperature sensor); NTCE (Elotec NTC water temperature sensor).

Measurement readings for non-NTC type sensors are calculated linearly between the programmed 'Max' and 'Min' values. In addition, the ‘Cal’ column allows you to enter a fine correction based on the following formula:

Reading =((Max - Min)1023×Measurement)+Min + Cal

NTC sensor readings are limited by the values entered in the 'Min' and 'Max' columns. An entry in the ‘Cal’ column produces a fine, though non-linear, correction differing slightly from the entered value.

refers to "arrosage" in French

aspi (suction) refers to the pump protection aspect.

1 a traversing system usually running on rails at some height above the ground